Supplementary Information for “Chasing wind and sun narrows but does not close the data-centre clean-power gap”

Junteng Liao1, Jiefu Lu2, Haoran Zhang1,*

1School of Urban Planning and Design, Peking University, Shenzhen, Guangdong, China

2School of Artificial Intelligence, Shenzhen Technology University, Shenzhen, Guangdong, China

*Corresponding author: Haoran Zhang (E-mail: h.zhang@pku.edu.cn)

Contents

S0. Overview of AI Workload Parameters

S1. Triangulated energy-basis calibration of the AI demand share and the inference-training split

S2. Empirical workload-shape fitting from BurstGPT v2

S3. Converting the burst-amplification operator from an empirical hourly burst to an additive amplification

S4. Independent cross-validation on the Azure LLM Inference Trace 2024

S5. Composite-workload mixing operator

S6. Summary of adopted AI-workload parameters

S7. Single-factor sensitivity sweep of the 2030 main results

S8. Country-level domestic mismatch data

S9. Supply-side capacity-factor construction and validation

S10. Cross-national supply portfolio optimisation

S11. Round-trip-time model and validation against measured Azure latencies

S12. Country sample, routing reach and cloud-region availability

S13. Storage cost conversion and duration-dependent storage economics

S14. Robustness of supply and routing assumptions

S15. Boundaries of deliverability and statistical convention

S16. Robustness of full electricity-supply cost to reliability target, annual cycling and storage-cost scenario

S17. The demand-shape model, compute-intensity weighting, and robustness to the flat-baseload share

S18. Scenario anchoring of the latency tolerance τ and the compute-to-network-budget conversion

S19. Positioning relative to prior work

S20. Independent external corroboration (E3)

S21. Identities linking the wind-and-solar matching gap U to established metrics

S22. Converting the phase-firming premium Pi to absolute levelised cost

S23. Literature basis and derivation of the two dimensionless measures

S24. Mathematical characterization and proof of the wind-and-solar phase floor

Supplementary References

This document is the Supplementary Information for the main paper Chasing wind and sun narrows but does not close the data-centre clean-power gap. Throughout, we adopt the framework used in the main text — the geographic lever routes compute load across longitudes over the internet to regions that currently hold a clean surplus (supply is fixed to the home country, with each country still using its own wind and solar), along two axes—latency tolerance τ (milliseconds, gating the reachable range by round-trip time) and routable fraction φ (the share of load that can be served off-site); the full electricity-supply cost comprises generation, firming storage and ancillary services, and excludes inter-regional transmission. Sections S0–S8 document the empirical fits, sensitivity analyses and cross-validation behind the AI workload parameters used in Result 3 (Methods (AI workload perturbation)); Sections S9–S14 document the supply-side and routing mechanisms underpinning Results 1–2 — the construction and validation of capacity factors, the mathematical formulation of home-country portfolio optimisation, the round-trip-time model validated against measured Azure latency, the country sample and cloud-region availability, the storage cost conversion, and the robustness of the supply and routing assumptions.

Supplementary Section S0 — Overview of AI Workload Parameters

In Result 3, every AI workload parameter is determined from public traces and forecasting evidence rather than by manual tuning. Table S0 indexes each parameter to its adopted value, its data source, and the supplementary section that provides the full method, confidence intervals, cross-validation and sensitivity.

Table S0. AI workload parameters, adopted values and data sources.

| Parameter (operator) | Adopted value | Data source | Suppl. section |
| --- | --- | --- | --- |
| λinf(t)— AI inference share (drives the P2/P3 shape operators); λtrain(t) — training, flat baseload | λinf0.130 / 0.308 / 0.560; λtrain 0.070 / 0.132 / 0.140 (2025 / 2030 / 2050) | Energy-based : Gartner, IEA-4E/EDNA, IEA (Energy and AI 2025); inference/training split: Google/Patterson 2022, IEA-4E/EDNA, EPRI 41,42,1,43,44 | S1 |
| μ, σ — P2 sharpening of the intraday peak | 18.0 h, 3.8 h | BurstGPT v2.0 (10.1 M requests / 235 observed days) 26 | S2 |
| 24-hour shape — P2_emp (no Gaussian assumption) | empirical token-share vector | BurstGPT v2.0 26 | S2.8 |
| γop— P3 burst amplification | 4.84 = (9.31 − 1) / 1.716 | BurstGPT hourly-aggregated P99/mean (9.31) ÷ Cloudflare /mean (1.716, 104 countries) 26,9 | S2, S3 |
| αbatch/ αshaped — Mix weights | 0.25 / 0.75 (flat batch processing / smoothed intraday-shaped inference; event bursts are isolated in P3 and not folded into Mix) | Carbon Responder (Xing et al. 2023, Meta), adapted to LLM serving 24 | S5 |
| Independent cross-validation | σconfirmed; γ shown to depend on service type | Azure LLM Inference 2024 (44 M requests) 27 | S4 |

The main results in Result 3 use the central values. Section S7 perturbs each parameter within its empirical range and confirms that the cost and gap spread from home-country (τ = 50 ms, φ = 0, no routing) to global (τ = 500 ms, φ = 100%, fully routable) is almost invariant to the AI shape parameters; a concise old-versus-revised comparison of these parameters is given in Section S6.

Supplementary Section S1 — Triangulated energy-basis calibration of the AI demand share and the inference–training split

S1.1 Motivation and rebasing

Result 3 reshapes a share λ of total data centre electricity demand using AI-workload operators (Methods (AI workload perturbation)). A consistent λ must satisfy two requirements that earlier single-parameter treatments failed to meet.

First, a unified measurement basis. Published AI-share figures conflate a capacity / power-demand basis (GW/GW, e.g. McKinsey, Goldman) with an energy basis (TWh/TWh, e.g. Gartner, IEA). Capacity shares are systematically higher than energy shares, because AI and accelerated servers run at higher and steadier utilization than conventional servers, so the two cannot be merged with equal weight. We therefore base λ on a single energy (TWh/TWh) basis, retaining capacity-basis sources only for the 2050 ceiling. 40,45,41,1

Second, inference and training are physically distinct loads. The shape operators P2 (sharpening) and P3 (bursting) impose an intraday, evening-peaked, event-bursty profile—an inference-serving phenomenon. At fleet scale, training is a sustained, near-flat, schedulable baseload (and, within the routing framework, also the load most readily relocated and served off-site). Reshaping the entire AI-related share with intraday and bursty operators would fold the flat training baseload into the evening peak, inflating the cost of the peaked scenarios. We therefore split λ into an inference component that drives the shape operators and a training component that enters as a flat baseload

λinf(t)=λAI(t) finf(t),  λtrain(t)=λAI(t) 1−finf(t)​	(S1)

where λAI is the total AI-related energy share and finf is the energy-basis inference fraction within AI. Only λinf is reshaped by P2/P3; λtrain enters each operator as a flat baseload term (Methods (AI workload perturbation)). The adopted total AI-related energy shares are 0.20, 0.44 and 0.70 for 2025, 2030 and 2050, respectively.

S1.2 Sources for the energy-basis AI demand share

Table S1. Energy-basis estimates of the AI demand share.

| Year | Source | Definition (energy basis) | λlow | λmid | λhigh |
| --- | --- | --- | --- | --- | --- |
| 2025 | Gartner (November 2025) 41 | AI-optimized servers / total data centre electricity (93/448 TWh) | 0.183 | 0.208 | 0.233 |
| 2030 | Gartner (November 2025) 41 | AI-optimized servers / total data centre electricity (432/980 TWh) | 0.416 | 0.441 | 0.466 |
| 2025 | IEA-4E / EDNA (2025) 42 | AI-related share of data centre electricity (5–15 % in 2023, growing at ~30 %/yr) | 0.120 | 0.169 | 0.210 |
| 2030 | IEA-4E / EDNA (2025) 42 | AI-related share of data centre electricity | 0.350 | 0.425 | 0.500 |

The two independent energy-basis sources agree closely for 2030 (Gartner 44.1 %, EDNA 35–50 %). The IEA Energy and AI report (2025) provides a plausibility check on the denominator — total data centre electricity rises from 415 TWh (2024) to about 945 TWh (2030), with accelerated servers accounting for nearly half of the net increment. Capacity-basis sources are excluded from λAI — McKinsey puts AI at ~70 % of data centre power demand (GW) in 2030, and Goldman at 39 % of data centre power—both higher than the energy share, and used only as a reference for the 2050 ceiling (S1.5). The energy-basis set (the equal-weight mean of the two sources) is λAI,mid = 0.19 (2025) and 0.43 (2030), rounded to the operating values λAI = 0.20 (2025) and 0.44 (2030). 41,42,1,40,45

S1.3 Inference fraction

finf is the energy-basis inference share of AI electricity (numerator = inference electricity; denominator = AI = training + inference). Only measured-fleet or review figures are used; the 80–90 % inference numbers circulating elsewhere are compute/FLOP or revenue splits (Schneider Electric, NVIDIA), not measured-fleet energy, and are therefore excluded. 43,42,44

Table S2. Measured inference fraction of AI electricity.

| Source | Inference / training (energy) | finf |
| --- | --- | --- |
| Google / Patterson et al. 2022 (measured fleet) 43 | 60 % / 40 % | 0.60 |
| IEA-4E / EDNA 2025 review 42 | 60–70 % / 20–40 % (+<10 % model development) | 0.60–0.70 |
| EPRI 44 | ~60 % / ~30 % (normalised 0.667) | ~0.67 |

The <10 % model-development overhead is folded into training (conservative with respect to deferrability), so that λinf + λtrain = λAI holds exactly. As deployment scales, the inference share rises — training is broadly amortized and one-off, whereas inference recurs throughout a model’s operating life and query volumes keep growing. We adopt finf = 0.65 (2025), 0.70 (2030) and 0.80 (2050).

S1.4 Decomposition into the Result 3 anchors

Table S3. Decomposition of λ into inference and training anchors.

| Year | λAI(mid) | finf | λinf(→ P2/P3) | λtrain(→ flat) |
| --- | --- | --- | --- | --- |
| 2025 | 0.20 | 0.65 | 0.130 | 0.070 |
| 2030 | 0.44 | 0.70 | 0.308 | 0.132 |
| 2050 | 0.70 | 0.80 | 0.560 | 0.140 |

TWh plausibility check on the central values — 2030 inference ≈ 0.308 × 945 ≈ 291 TWh; 2030 training ≈ 0.132 × 945 ≈ 125 TWh. The 2030 inference range (low/high) is [0.21, 0.375] (from λAI [0.35, 0.50] × finf [0.60, 0.75]), used in the sensitivity sweep of S7.

S1.5 Logistic extrapolation and the 2050 ceiling

The anchoring data extend only to 2030, so the saturation level is an explicit prior. The total AI energy share tends to 0.70 in 2050 (range 0.60–0.80) — conventional and cloud workloads keep growing, so AI cannot approach 100 % of data centre energy, but over decades AI grows faster than they do (McKinsey puts AI at ~70 % of data centre GW in 2030, lower once converted to an energy basis; ABI puts AI at ~64 % of data centre energy in 2035 and still rising). finf rises to 0.80 (range 0.70–0.85). The logistic fit for λAI is 40,46

λAI(t)=0.02+0.70−0.021+exp​−k (t−t0)​	(S2)

with k ≈ 0.31 and t0 ≈ 2028.6, giving λAI,2050 = 0.699 ≈ the ceiling. Combined with finf,2050 = 0.80, this yields λinf,2050 = 0.560 and λtrain,2050 = 0.140.

Figure S1 | Triangulated energy-basis AI demand share and its inference/training split, 2024–2050. (a) The AI share of data-centre electricity on an energy basis, λAI, follows a logistic trajectory (floor 0.02, ceiling 0.70, k = 0.31, t0 = 2028.6; solid line) anchored to the ensemble mid-points λAI = 0.20 (2025), 0.44 (2030) and 0.70 (2050) (filled circles); vertical bars give the source ranges (2030 [0.35, 0.50]; 2050 [0.60, 0.80]), and the individual Gartner and IEA-4E/EDNA estimates are shown for 2025 and 2030. (b) Decomposition of λAI into the inference share λinf (which drives the P2/P3 intraday shape operators) and the flat training baseload λtrain, as stacked areas summing to λAI; the dashed line is the inference fraction finf (right axis), rising from 0.65 to 0.80. The anchor values are λinf = 0.130 / 0.308 / 0.560 and λtrain = 0.070 / 0.132 / 0.140 for 2025 / 2030 / 2050 (Table S3).

Supplementary Section S2 — Empirical workload-shape fitting from BurstGPT v2

S2.1 Data

We use the public BurstGPT v2.0 trace (Wang et al. 2024, arXiv — 2401.17644). The cleaned (failure-free) release contains three trace windows. 26

Table S4. Cleaned BurstGPT v2.0 trace windows.

| Trace window | Requests (cleaned) | Time span | Notes |
| --- | --- | --- | --- |
| Window 1 | 1,404,294 | ~61 days | First trace window |
| Window 2 | 3,784,213 | ~63 days | Second window |
| Window 3 | 4,956,058 | ~111 days | Third (longest) window |
| Total | 10,144,565 | ~235 observed days | Pooled fit |

Timestamps are given in seconds from 00:00 on day 1 and are calibrated to the user’s local time zone (per the BurstGPT v2.0 documentation), so the hour-of-day directly indexes the user’s local hour.

S2.2 Methods

For each weighting w ∈ {request count, total tokens, response tokens}, we (1) aggregate w by hour-of-day = (timestamp // 3600) mod 24 and normalize to sum-to-one shares, yielding the intraday share profile; (2) fit a Gaussian by nonlinear least squares,

g(h)=base+amp⋅exp​−(h−μ)22σ2​	(S3)

and (3) compute burst quantiles on 1-hour binned sums, reporting the mean-relative P50, P90, P95, P99, P999 and maximum.

S2.3 Pooled fit

Table S5. Pooled Gaussian fits by mass weighting.

| Weighting | Peak hour μ | σ(h) | FWHM (h) |
| --- | --- | --- | --- |
| Request count | 18.31 | 3.99 | 9.40 |
| Total tokens (primary) | 18.05 | 3.79 | 8.93 |
| Response tokens (compute proxy) | 18.12 | 3.98 | 9.37 |

The three weightings give virtually identical peaks (within 0.3 h of each other) and σ values within 0.2 h, indicating that the intraday structure is robust to the choice of mass proxy.

S2.4 Bootstrap 95% confidence intervals

We resample 5-minute token-volume bins with replacement (500 iterations), refit the Gaussian for each resample, and report the 2.5 / 50 / 97.5 percentiles. Only the total-token weighting (primary) is shown.

Table S6. Bootstrap 95% confidence intervals (total-token weighting).

| Parameter | Point estimate | 95% CI (2.5–97.5 percentile) |
| --- | --- | --- |
| Peak hour μ | 18.05 | [17.75, 18.39] |
| σ(h) | 3.79 | [3.53, 4.05] |

The confidence intervals are narrow — the intraday AI-inference peak is pinned to within ±0.3 h and σ to within ±0.3 h.

Figure S2 | Empirical intraday inference shape, Gaussian fit and bootstrap confidence interval. (a) The normalised hour-of-day token share pooled over the BurstGPT v2 trace (10.1 M requests; grey bars) is bimodal, peaking at 0.064 at hour 17 with a secondary lobe at hours 19–20. The fitted Gaussian (token weighting, peak μ = 18.0 h, σ = 3.8 h; purple line) and the 95% bootstrap confidence band (500 resamples; shaded) are overlaid, with the dashed line marking μ = 18.0 h. (b) Stationarity of the fit across the three trace slices in (μ, σ) space (orange circles, with each slice’s hourly P99/mean labelled) against the pooled token fit (teal diamond; P99/mean = 9.31) and the 95% bootstrap confidence rectangle (μ ∈ [17.75, 18.39], σ ∈ [3.53, 4.05]). The peak hour and width are stable across slices; only the burst ratio varies, motivating the γ sensitivity range used in S7.

S2.5 Temporal stationarity

To test whether the intraday shape drifts over the ~11-month trace, we refit each window slice separately.

Table S7. Stationarity of the intraday fit across trace slices.

| Slice | n requests | Duration | Peak hour | σ(h) | Hourly P99/mean |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.40 M | 61 d | 17.69 | 4.20 | 10.63 |
| 2 | 3.78 M | 63 d | 17.07 | 4.20 | 5.07 |
| 3 | 4.96 M | 111 d | 18.38 | 3.26 | 10.70 |

Peak hour and σ are stable across slices (peak ∈ [17.1, 18.4], σ ∈ [3.3, 4.2]). The only parameter with appreciable variation is the hourly P99/mean ratio (slice 2 is about 2× quieter in bursts than slices 1 and 3). We take the pooled estimate as the central value and fold this within-trace variability into the γ sensitivity range in S7.

S2.6 Derivation of the burst-amplification operator

The BurstGPT hourly P99 / mean ratio (pooled, token) is 9.31, with a per-slice range of [5.07, 10.70]. It is mapped through Section S3 to the P3 operator γ.

S2.7 Slice-2 burst-ratio outlier

The hourly P99/mean for slice 2 (5.07) is roughly half that of slice 1 (10.63) and slice 3 (10.70). Investigation shows that this is not a parameter-instability issue in the Gaussian fit—the peak hour and σ are stable (17.07 and 4.20 for slice 2 versus 17.69 / 4.20 for slice 1 and 18.38 / 3.26 for slice 3)—but rather a difference in traffic composition. The calendar window covered by slice 2 contains fewer extreme viral spikes in the trace, with a more uniform day-to-day variance. We treat slice 2 as the low-burst end of the empirical within-trace range and slices 1 and 3 as the high-burst end, with the pooled estimate (γemp = 9.31) lying between them. This within-trace variability is precisely what motivates the choices γ ∈ {3.0, 4.84, 6.0} in S7—it spans both the stylized prior and an upper bound consistent with the high-burst slices.

S2.8 Empirical-shape operator versus Gaussian-sharpening operator

The Gaussian obtained from the BurstGPT fit (μ = 18.0, σ = 3.8), although convenient, smooths over a real bimodal structure in the empirical 24-hour profile — the token share peaks at hour 17 (0.064) and again at hours 19–20 (0.058–0.059), with a slight midday dip, and retains substantial mass (> 0.04) throughout hours 12–23. To remove the Gaussian assumption from the chain, we introduce the empirical-shape operator P2_emp

dc(P2emp)(t)=(1−λinf−λtrain) dc(t)+λinf semp(h(t)) ⟨dc⟩+λtrain ⟨dc⟩​	(S4)

where semp is the pooled BurstGPT v2 24-hour share vector, normalised to mean = 1 (a parameter-free assumption); only the inference share λinf takes the empirical shape, while the training share λtrain is a flat baseload (S1). P2_emp is one of the four deferred operators shown in Fig. S4 (main-text Fig. 4 and Fig. 5 use only the realistic Mix as the representative case; the remaining four operators—batch-plus-training, consumer chat, measured chat and viral spike—together with the AI-free baseline Today as a reference block, are all deferred to Fig. S4).

Table S8. Empirical-shape operator versus Gaussian-sharpening operator in 2030 (national-level Result 3 over 104 countries, τ×φ basis, full electricity-supply cost).

| Operator | Domestic uncovered | Domestic cost (USD/MWh) | Global uncovered | Global cost (USD/MWh) |
| --- | --- | --- | --- | --- |
| P2 sharpened (Gaussian) | 34.5 % | 126.6 | 14.3 % | 114.4 |
| P2_emp (BurstGPT raw 24-hour) | 31.5 % | 120.7 | 13.2 % | 108.5 |
| Δ(Gaussian relative to empirical) | +3.0 pp | +5.9 | +1.1 pp | +5.9 |

Domestic is the τ=50 ms, φ=0 (no routing) corner, and global is the τ=500 ms, φ=100% (fully routable) corner. The Gaussian abstraction overstates the global-tier full electricity-supply cost by about 5% (114.4 vs 108.5 USD/MWh), with the most affected firming-storage component overstated by about 19% (37.6 vs 31.6), because it concentrates the inference mass on a single sharp peak (peak height ≈ 1.7 × mean), whereas the empirical shape is broader and multi-peaked, allowing more AI load to overlap with available renewable supply. P2_emp is methodologically cleaner and is the recommended reference; we report both because (i) the Gaussian P2 is widely understood and easy to perturb (CI bounds, alternative μ, σ) and (ii) showing both quantifies the bias introduced by this parametric simplification. Under the rebasing (only λinf is reshaped, λtrain flat), the six AI shapes at the global tier rank in ascending full electricity-supply cost as batch-plus-training (107) < measured chat (108.5) < Mix (112.3) < viral spike (112.7) < AI-free baseline (114.1) < consumer chat (114.4); the six differ by only about 7 USD/MWh, far smaller than the domestic-to-global routing drop (about 12 USD/MWh)—the burst spike, because it concentrates only in the top-1% hours and is flatter the rest of the time, is in fact more readily absorbed by routing and storage than consumer chat, which sustainedly raises the evening peak.

Supplementary Section S3 — Converting the burst-amplification operator from an empirical hourly burst to an additive amplification

S3.1 Derivation

The P3 burst-amplification operator in the main text is

d(P3)(t)=(1−λtrain) d(t)+λtrain ⟨dc⟩+1​d(t)≥Q0.99(d) d(t) λinf γop​	(S5)

That is, bursts (an inference phenomenon) scale with the inference share λinf, whereas the training share λtrain is flattened into a baseload (S1). At the top-1% hours, the burst excess injected by AI equals d(t) · λinf · γop. The BurstGPT trace yields the empirical burst ratio of the AI workload’s own time series, γemp ≡ hourly P99 / hourly mean. To express the operator parameter γop in terms of these quantities, we equate the excess — the inference mass at a burst hour should equal γemp times its own mean. Under equal-energy alignment (mean inference load = λinf · meand), this gives

γop=γemp−1dburst/meand​	(S6)

where dburst / meand is the peak-to-mean ratio of human traffic at the top-1%.

S3.2 Country-level burst ratios from Cloudflare

We computed dburst / mean individually for each of the 104 demand countries from the Cloudflare Radar hourly human-traffic proxy (2025-05-02 to 2026-05-01).

Table S9. Cross-country distribution of the human-traffic peak-to-mean ratio.

| Statistic | dburst/ mean |
| --- | --- |
| Minimum | 1.353 |
| 25th percentile | 1.632 |
| Median (recommended) | 1.716 |
| Mean | 1.777 |
| 75th percentile | 1.885 |
| Maximum | 2.950 |

The cross-country median of 1.716 is adopted as the calibration value for the human-traffic burst-to-mean ratio.

S3.3 Recommended burst-amplification coefficient

Combining S2.6 (γemp = 9.31) with S3.2 (median dburst/mean = 1.72) yields γop = (9.31 − 1) / 1.72 = 4.84, with a country-level IQR-derived range of [4.41, 5.09]. This mapping calibrates the P3 amplification directly to the BurstGPT burst ratio and the observed cross-country variation.

Figure S3 | Human-traffic peak-to-mean ratios and the derived burst-amplification coefficient. (a) Distribution across the 104 demand countries of the hourly human-traffic peak-to-mean ratio dburst/mean (Cloudflare Radar, 2025-05-02 to 2026-05-01); the cross-country median is 1.716 (orange line), the interquartile range [1.632, 1.885] is shaded, and the full range spans [1.353, 2.950]. This median is used to calibrate the P3 coefficient. (b) Mapping to the P3 burst-amplification coefficient γop=(γemp−1)/(dburst/mean) with γemp = 9.31 (pooled BurstGPT hourly P99/mean) — the recommended default is γop = 4.84 at the median ratio (circle), with the IQR-derived band [4.41, 5.09] shaded.

Supplementary Section S4 — Independent cross-validation on the Azure LLM Inference Trace 2024

S4.1 Dataset

The Azure LLM Inference Dataset 2024 is an independent enterprise-grade LLM-serving trace, comprising two service types and collected from 10–19 May 2024. 27

Table S10. Azure LLM Inference 2024 service traces.

| Service | Requests | Span | UTC start | UTC end |
| --- | --- | --- | --- | --- |
| conv (conversational) | 27,303,835 | 7.0 days | 2024-05-12 00:00 | 2024-05-19 00:00 |
| code (code assistant) | 16,803,695 | 7.0 days | 2024-05-10 00:00 | 2024-05-17 00:00 |
| Total | 44,107,530 |  |  |  |

Each record contains a timestamp, an input (context) token count, and a generated token count; timestamps are absolute UTC date-times (unlike BurstGPT, no local-timezone calibration was applied).

S4.2 Fitting and cross-validation

Table S11. Diurnal fits and burst ratios for BurstGPT and Azure.

| Trace | Service | Peak (h) | σ(h) | Hourly P99/mean |
| --- | --- | --- | --- | --- |
| BurstGPT v2 merged | (all, local timezone) | 18.05 | 3.79 | 9.31 |
| Azure LLM 2024 | conv (UTC) | 13.90 (counts) / 10.6 (tokens, noisy) | 3.43 (counts) | 1.53 |
| Azure LLM 2024 | code (UTC) | 17.92 | 3.24 | 3.10 |
| Azure LLM 2024 | merged (tokens) | 17.22 (UTC) | 3.17 | — |

Three findings are noteworthy. First, σ is consistent across the two datasets (BurstGPT 3.79 h, Azure merged 3.17 h, both within the [3.0, 4.0] h range), corroborating the roughly 9 h FWHM diurnal peak of AI inference. Second, the interpretation of the peak hour varies with the timezone reference — Azure’s 17 UTC peak corresponds to European evening (19:00 local) or US-Eastern midday (13:00 EST), which, when the user population is weighted toward Europe and the United States, is broadly consistent with BurstGPT’s 18 (local timezone) peak. Third, γemp varies by service type — BurstGPT consumer chat (9.31) ≫ Azure code (3.10) > Azure enterprise conv (1.53) — consumer-facing chat, subject to viral and news spikes, is far burstier than enterprise inference services. Result 3 covers this range through the S7 sensitivity over γ ∈ {3.0, 4.84, 6.0}. The independent Azure trace corroborates the BurstGPT diurnal shape but reveals that burst intensity depends on service composition, validating both the empirical γ default (BurstGPT, the consumer-facing worst case) and the necessity of the composite Mix operator (Supplementary Section S5).

Supplementary Section S5 — Composite-workload mixing operator

S5.1 Motivation

Real AI inference is not a single workload type. It spans batch and offline work (embeddings, content moderation, evaluation, fine-tuning jobs), which is latency-tolerant and naturally flat over 24 hours; and interactive and event-driven services (chat, copilots, voice, together with the viral and news spikes layered on top), which follow a BurstGPT-shaped diurnal distribution. The single-operator runs P1, P2 and P3 in the main text each isolate the effect of one shape in isolation. The composite Mix operator combines a flat batch share with a follow-the-sun share carrying a smooth BurstGPT evening shape. Top-1% burst amplification is deliberately not folded into the main-report Mix — concentrating all event-driven load into a single peak hour would inject the extra energy directly into the post-sunset gap window and would raise the central case on account of a single hour; event-driven bursts are therefore isolated in the separate P3 operator, whose contribution is reported on its own.

S5.2 Mix weights

Table S12. Composite Mix operator weights.

| Component | Weight α | Basis |
| --- | --- | --- |
| Batch (flat) | 0.25 | The OpenAI Batch API (50% discount, 24 h SLA) implies that scheduled inference is roughly ~20-30% of serving capacity; EcoServe reports that offline/batch makes up a large share of generative-AI inference 76,78 |
| Follow-the-sun (BurstGPT shape) | 0.75 | Combines interactive services (chat, code copilots, RAG; about 0.65, the highest-revenue products that drive most inference traffic) with event-driven services (viral, news and release spikes; about 0.10, a tail phenomenon reasonably estimated at 5-15% of total serving). Both are represented by the same smooth BurstGPT diurnal shape; top-1% burst amplification is not applied here but isolated in the separate P3 operator 26,24 |

The defaults sum to 1.0. The Azure LLM trace (S4) supports treating most serving as diurnally shaped — its enterprise conv (γ ≈ 1.5) is closer to flat-shape serving, whereas consumer-facing chat (BurstGPT γ ≈ 9.3) is strongly diurnally concentrated. The main-report Mix therefore carries a smooth diurnal shape without a single-hour spike; the burst tail is quantified separately by P3.

S5.3 Operator definition

d(Pmix)(t)=(1−λinf−λtrain) d(t)+λtrain⟨d⟩+λinfαbatch⟨d⟩+αshaped wemp(h(t))⟨d⟩​	(S7)

where wemp(h) is the Gaussian fitted to BurstGPT v2 (μ = 18.0, σ = 3.8, normalised to mean = 1), αbatch = 0.25 and αshaped = 0.75. The composite AI mix acts on the inference share λinf; the training share λtrain enters as a flat baseload (S1). Note that the αbatch term (flat, scheduled inference) is conceptually distinct from λtrain (model training), even though both are flat. The main-report Mix contains no top-1% amplification term; each component preserves the mean of the inference share, so the operator adds no net energy and injects no single-hour spike into the evening gap window. Event-driven bursts are quantified separately by the P3 operator (γop = 4.84; Supplementary Section S3).

S5.4 Sensitivity

With event bursts isolated in P3, the only remaining degree of freedom in Mix is the flat batch share (the rest follows the smooth diurnal shape). We report the residual uncovered share and the full electricity-supply cost for the 2030 global tier (τ = 500 ms, φ = 100%) under three plausible batch shares, taken from the single-factor sweep in S7.

Table S13. Sensitivity of the 2030 global-tier residual to the batch (flat) share.

| Batch (flat) share | Global-tier uncovered share | Global-tier full electricity-supply cost (USD/MWh) |
| --- | --- | --- |
| 0.15 (sharper, more diurnally shaped) | 14.0% | 112.9 |
| 0.25 (default) | 13.9% | 112.3 |
| 0.35 (flatter, more batch) | 13.7% | 111.5 |

A flatter batch share is slightly cheaper, but the range is only about ±1 USD/MWh around the default, confirming that the Mix split has no material impact on the main results (full sweep in S7).

S5.5 Presentation of the Mix operator in the main text

Mix is the only operator shown as a representative in the main text — it runs through all panels of Figure 4 (mechanism) and Figure 5 (cost)—in Figure 4, (a) the three-layer intraday demand decomposition, (b) the zero-sum gap panel over clean supply, (c) the six-shape domestic vs global uncovered-share dumbbell plot and (d) the uncovered-share heatmap of routable fraction × latency tolerance; in Figure 5, (a) the waterfall in which cost steps down as latency is relaxed (the global tier being the lowest step), (b) the rose diagram of full electricity-supply cost across 36 scenarios, (c) the ridgeline of year-by-year cost distributions for the domestic and global tiers and (d) the world map of country-by-country full electricity-supply cost. The main-text narrative cites Mix as the realistic expected case that lies between the two extremes—batch and training (flattest, cheapest) and consumer chat (sharpest evening peak, most expensive).

S5.6 The four non-main-figure AI operators

Main-text Figures 4 and 5 use only the realistic mix (Mix) as the representative. The four deferred operators—together with the AI-free baseline (Today) as a reference block—are shown here in the same τ×φ framing and the same colour scheme as main-text Figures 4 and 5, to allow direct comparison with the main-text figures.

Figure S4 | Residual gap and cost are insensitive to AI demand shape. The operators comprise the AI-free baseline (P0, labelled Today in the figure) plus four deferred operators—batch and training (P1, flat/dispatchable), measured chat (P2_emp, empirical bimodal), consumer chat (P2, evening Gaussian peak) and viral event spikes (P3, top-1% amplification). (a)-(e) One block per operator — above, the intraday demand curve for that shape (2025/2030/2050 overlaid on the AI-free grey dotted line, equal-energy normalised, mean = 1); below, the 2030 heatmap of uncovered demand share over latency tolerance (columns, 50→500 ms) × routable fraction (rows, 0→100%) (cell colour and printed value encode the same share, %; layout as in main-text Figs 2a and 4d). (f) Cost view — dumbbell plot of the median full electricity-supply cost across 104 countries (USD/MWh, 2030) for the six shapes (including Mix) under the domestically reachable tier (50 ms, no routing) and the globally reachable tier (500 ms, fully routable). For every operator, the collapse of uncovered share and cost along the latency/routability axes far exceeds the differences produced by the shape itself — the six shapes differ by only about 7 USD/MWh within each tier, whereas the domestic-to-global routing drop is about 30; the cost ranking of the shapes is given in S2.8.

Supplementary Section S6 — Summary of adopted AI-workload parameters

Table S14 consolidates the AI-workload parameters adopted in the analysis and their empirical basis. The total AI-related energy share is split into inference, which drives the shape operators, and training, which enters as a flat baseload. The adopted central values are listed here; uncertainty ranges and single-factor sensitivity are evaluated in Supplementary Section S7.

Table S14. Adopted AI-workload parameters and empirical sources.

| Parameter | Adopted value | Source |
| --- | --- | --- |
| λinf,{2025} | 0.130 [low 0.090, high 0.175] | λAI0.20 × finf 0.65 (S1) |
| λinf,{2030} | 0.308 [low 0.210, high 0.375] | λAI0.44 × finf 0.70 (S1) |
| λinf,{2050} | 0.560 | λAI0.70 × finf 0.80 (S1) |
| λtrain,{2025/2030/2050} | 0.070 / 0.132 / 0.140 | λAI× (1 − finf); flat baseload (S1) |
| P2 peak μ | 18.0 h [17.75, 18.39] | BurstGPT v2 pooled fit (S2) |
| P2 σ | 3.8 h [3.53, 4.05] | BurstGPT v2 pooled fit (S2) |
| P3 γop | 4.84 [4.41, 5.09] | BurstGPT v2 pooled P99/mean (9.31) ÷ country-by-country dburst/mean (1.716) (S2, S3) |
| dburst/ mean | 1.716 [1.63, 1.89] | Cloudflare 104-country median (S3) |
| Mix weights (batch / diurnally shaped) | 0.25 / 0.75 | Literature-anchored AI service mix; event bursts isolated in P3 (S5) |
| Cross-validation | Azure LLM 2024 (44M requests) confirms σ and frames γ by service type | S4 |

Supplementary Section S7 — Single-factor sensitivity sweep of the 2030 main results

S7.1 Design

For the representative year 2030, we perturb each AI-workload parameter within its empirical uncertainty interval (holding the others at their central values) and re-solve the country-level Result 3 for 104 countries at the two corner cases — domestic (τ=50 ms, φ=0, no routing) and global (τ=500 ms, φ=100%, fully routable). The reported quantities are the 104-country median full electricity-supply cost (LCOE, USD/MWh) and the residual uncovered share (%). Each parameter is applied to its most sensitive operator — λinf and γ act on the P3 burst, (μ,σ) on the P2 sharpening, and the batch share on the Mix (script run_r3_latency_sensitivity.py).

S7.2 Sensitivity table

We evaluate the following variants on the full 104-country, country-level Result 3 portfolio.

Table S15. Perturbation variants and their empirical sources.

| Parameter | Variant | Source |
| --- | --- | --- |
| λinf,{2030} | low 0.21 / mid 0.308 / high 0.375 | energy-basis inference interval (S1) |
| P2 (μ, σ) | CI low (17.75, 3.53) / mid (18.0, 3.79) / CI high (18.39, 4.05) | BurstGPT bootstrap 95% CI (S2) |
| P3 γ | 3.0 stylized / 4.84 default / 6.0 upper bound | covers stylized prior + high-burst slice (S2–S3) |
| Mix batch share (αbatch) | 0.15 / 0.25 default / 0.35 (remainder = smooth daytime shape) | literature mixing bounds (S5) |

Table S16. Domestic and global full electricity-supply cost (USD/MWh) and global-tier uncovered share (%) under single-factor perturbations for 2030, 104-country median.

| Parameter | Variant | Domestic cost | Global cost | Global uncovered | Global cost spread |
| --- | --- | --- | --- | --- | --- |
| λinf,{2030} (applied to P3 burst) | low (0.21) | 123.3 | 113.6 | 14.0 % | — |
|  | mid (0.308) | 123.9 | 112.7 | 14.3 % | central |
|  | high (0.375) | 124.1 | 112.2 | 14.5 % | ±0.8 % |
| P2 (μ, σ) bootstrap CI (applied to P2 sharpening) | CI low (17.75, 3.53) | 126.7 | 114.5 | 14.4 % | — |
|  | CI mid (18.0, 3.79) | 126.6 | 114.4 | 14.4 % | central |
|  | CI high (18.39, 4.05) | 126.8 | 114.5 | 14.6 % | ±0.1 % |
| P3 γ (applied to P3 burst) | stylized (3.0) | 123.4 | 113.6 | 13.9 % | — |
|  | default (4.84) | 123.9 | 112.7 | 14.3 % | central |
|  | upper bound (6.0) | 124.2 | 112.3 | 14.5 % | ±0.8 % |
| Mix batch share (applied to Mix) | default (0.25) | 124.9 | 112.3 | 13.9 % | central |
|  | flatter (0.35) | 124.2 | 111.5 | 13.7 % | — |
|  | sharper (0.15) | 126.2 | 112.9 | 14.0 % | ±0.5 % |

S7.3 Key observations

Every sensitivity interval preserves the domestic-to-global drop in cost and gap — across all 12 variants the domestic full electricity-supply cost stays within [123.3, 126.8] and the global cost within [111.5, 114.5] USD/MWh, with the global uncovered share within [13.7, 14.6] %—the roughly 12 USD/MWh (about −19 percentage points) routing drop from domestic to global is almost invariant to any AI parameter, with the latency tolerance × routable fraction dual-axis lever overwhelmingly dominant. Each AI parameter moves global cost little (spread ≤ ±0.8 %) — the λinf interval and P3 γ are slightly larger (about ±0.8 %), the Mix batch share next (about ±0.5 %), and the P2 CI negligible (±0.1 %); because the central λinf,{2030} = 0.308 is the main-result value, the λinf-mid row coincides exactly with the γ-default row that is likewise applied to P3 (domestic 123.9, global 112.7), confirming internal consistency between the sweep and the main results. The P2 bootstrap CI is already narrow enough to be ignored at the main-result level — across the entire 95 % CI the global cost barely moves (114.4–114.5). Finally, the bias introduced by the parameterized Gaussian abstraction in P2 exceeds the within-Gaussian parameter uncertainty (S2.8 — global-tier P2_emp 108.5 vs Gaussian P2 114.4, ≈ +5 %), which supports reporting both, using P2_emp as the methodologically cleaner reference while retaining the Gaussian P2 for its analytical tractability under perturbation.

S7.4 Factors not covered

A full multi-factor Monte Carlo (jointly sampling λ, μ, σ, γ and the mixing weights) would require about 500 Result 3 re-runs. The single-factor sweep characterises local sensitivity but does not capture joint parameter uncertainty; a multivariate analysis could produce either a narrower or wider envelope.

Supplementary Section S8 | Country-level domestic mismatch data

Under the equal-energy domestic accounting of Result 1 (Methods (hourly panel, equal-energy normalisation and domestic mismatch metrics)), Supplementary Data 1 reports for each of the 104 sampled countries the annual hourly wind-and-solar matching gap U (i.e. the annual uncovered-demand share; its identities with the 24/7 CFE score, temporal self-sufficiency, load match index and overlap coefficient are given in S21), the peak local hours of demand and supply and their phase lag, continuous-gap-segment statistics (median, 95th percentile and maximum segment length, hours), the maximum hourly uncovered intensity relative to mean demand, and the intraday spectral concentration of demand and supply. The country-by-country data are supplied separately as Supplementary Data 1.

Supplementary Section S9 — Supply-side capacity-factor construction and validation

S9.1 Construction

The supply side is built from real wind and solar PV power stations rather than a single national representative capacity factor (Methods (hourly panel and equal-energy normalisation)). From the WRI Global Power Plant Database we took all 16,009 solar PV and wind units (latitude, longitude, installed capacity) and aggregated them on a 3° grid into 455 capacity-weighted representative stations, retaining about 91 % of global installed capacity and covering every country on record. The three demand countries with no domestic stations on record (Indonesia, Libya, Paraguay) each received a national-centroid pairing (one solar PV, one wind), for 461 candidate stations in total. For each station we drew hourly shortwave irradiance, 2 m air temperature and 100 m wind speed from the Open-Meteo ERA5 archive for 2025-05-02 to 2026-05-01, aligned with the demand window. 10,37,38

Solar PV capacity factors follow the first-order PVWatts/GSEE model,

CFPV=clip​G1000[1+γ (Tcell−25)], 0, 1​	(S8)

Tcell=T2m+NOCT−20800 G​	(S9)

with NOCT = 45 °C and γ = −0.0035 ​∘C−1. Wind capacity factors use a smoothed IEC single-turbine power curve (cut-in 3.5, rated 12, cut-out 25 m/s), with the wind-speed distribution Gaussian-smoothed to approximate multi-turbine aggregation; because reanalysis systematically underestimates output at high-quality wind sites (Davidson & Millstein 2022), a ×1.2 bias correction is applied to the wind capacity factor. National solar PV and wind capacity factors are the domestic capacity-weighted means of the respective station factors; cross-national weighting uses IRENA grid-connected solar PV / wind installed capacity.

S9.2 Validation

Three lines of evidence delimit the realism of the constructed factors. First, equal-energy normalisation makes the results insensitive to level — demand and supply are each rescaled to the same annual total (Methods (hourly panel and equal-energy normalisation)), so any residual mismatch reflects only differences in temporal shape, not the absolute level of the capacity factors. Sweeping the wind ×1.2 correction between 1.0 and 1.4 changes only the absolute level of the wind capacity factor, not its temporal shape, so the portfolio results are essentially unchanged (Methods (robustness and sensitivity)). Second, coverage is not a binding choice — lowering the capacity coverage of the 3° grid representative stations from the default ~91 % to ~80 % or raising it to ~96 % shifts the global-reach median uncovered share by less than 0.5 pp (Methods (robustness and sensitivity)). Third, the methodology is well established — the reanalysis-to-capacity-factor chain (PVWatts/GSEE solar PV model, IEC wind power curve on ERA5) is the same validation approach used by Pfenninger & Staffell (2016) and Tong et al. (2021), and the resulting national mean capacity factors fall within the published ranges for utility-scale solar PV (~10–25 %) and onshore wind (~20–45 %) reported by IRENA and the IEA.

We further validate the modelled capacity factors directly against measured hourly generation, at the level of timing rather than annual mean. Two countries with open, hourly measured generation and representing distinct regimes were chosen — Germany (SMARD / Bundesnetzagentur; continental, solar and onshore wind) and Great Britain (Elexon / BMRS; maritime, offshore-wind-dominated) — and over the paper’s own window (2 May 2025 to 1 May 2026) their measured hourly solar/wind generation was aligned hour-by-hour with our modelled national capacity factors (each normalised to unit mean, so that only timing is compared), yielding the hourly correlation, diurnal-profile correlation, peak-hour offset and seasonal correlation of Table S17. For solar, the hourly correlation is 0.94 in both countries, the diurnal-profile correlation 0.96, the peak-hour offset ≤ 1 h and the seasonal correlation 0.99; for wind, the hourly correlation reaches 0.94 in offshore-dominated Great Britain and is 0.68 in onshore, complex-terrain Germany (a known limitation of reanalysis for onshore-wind sub-daily variability), while its diurnal and seasonal structure still correlate at 0.91 and 0.95. The phase structure on which this study depends — sunset timing, diurnal shape and seasonality — is thus directly supported by measured generation; wind–solar complementarity and continuous-deficit duration are built on these validated timing features rather than on annual-mean energy.

Table S17. Timing validation of the modelled capacity factors against measured hourly generation (2 May 2025 – 1 May 2026, shapes normalised to unit mean).

| Country · tech | Measured source | Hourly r | Diurnal r | Peak-hour offset | Seasonal r |
| --- | --- | --- | --- | --- | --- |
| Germany · solar PV | SMARD | 0.94 | 0.96 | +1 h | 0.99 |
| Germany · wind | SMARD | 0.68 | 0.91 | +2 h | 0.95 |
| Great Britain · solar PV | Elexon | 0.94 | 0.96 | +1 h | 0.99 |
| Great Britain · wind | Elexon | 0.94 | 0.93 | −1 h | 0.98 |

S9.3 Robustness to the capacity-database vintage

The cross-country wind:solar weight used to blend each country’s home supply is taken from IRENA 2025 grid-connected capacity, giving the reported median matching gap U of 40.1%; the WRI database enters only through station coordinates, which fix the resource shape via reanalysis meteorology, and the within-country capacity distribution. Recomputing the same median entirely on the older WRI-2021 capacity instead gives 44.7%, so the 2021 vintage would raise, not lower, the estimated mismatch, and the wind-share–U gradient (Fig. 1d) stays strongly negative under either vintage (Spearman ρ ≈ −0.7). The headline portfolio and routing gaps (41.8% and 12.7%) are computed on an equal-energy station-level basis that carries no fixed installed ratio and are invariant to the capacity vintage by construction. The only element country totals cannot refresh is the geographic location of stations built after 2021; because new solar sites share the noon-locked diurnal shape of existing ones and only offshore wind, a small share of global additions, materially changes supply timing, this cannot displace the evening-deficit structure that sets U.

S9.4 Interannual robustness of the matching gap

Because U is an equal-energy shape metric, interannual weather variability — chiefly a magnitude effect on annual energy — should divide out. We test this on eight representative countries spanning the gap range and both supply mixes (US, DE, GB, SA, IN, AU, BR, ZA). Holding each country's demand shape fixed, we rebuild its wind-and-solar supply from seven consecutive May–April windows of Open-Meteo ERA5 reanalysis (2019/20 through 2025/26) using the identical construction as the main analysis — WRI-capacity-weighted national PV and wind capacity factors combined by IRENA installed capacity (Section S9.1) — and recompute U for each year. The 2025/26 window reproduces the headline per-country gap to within 0.01 percentage points, so the interannual spread below is on the exact paper basis.

Table S18. Interannual robustness of the hourly wind-and-solar matching gap across representative countries (seven May–April windows, 2019/20–2025/26).

| Country | Dominant supply | Interannual range (pp) | Std (pp) | 2025/26 U (%) |
| --- | --- | --- | --- | --- |
| United States | wind+solar | 0.7 | 0.3 | 16.8 |
| Germany | wind-heavy | 2.7 | 0.9 | 22.6 |
| Great Britain | wind-heavy | 2.6 | 1.0 | 24.5 |
| Saudi Arabia | solar | 0.6 | 0.2 | 54.3 |
| India | solar-dominant | 1.0 | 0.3 | 29.9 |
| Australia | solar+wind | 1.6 | 0.5 | 25.9 |
| Brazil | wind+solar | 1.3 | 0.5 | 23.9 |
| South Africa | solar+wind | 1.3 | 0.4 | 31.0 |

The matching gap is highly stable across years (table below): the median country's interannual range over the seven years is 1.3 percentage points and the mean standard deviation 0.5 points. Solar-dominated grids are essentially invariant (Saudi Arabia 0.6, the United States 0.7, India 1.0 points of range), while wind-heavy grids show the largest but still modest spread (Germany 2.7, Great Britain 2.6 points, the widest single deviation in the low-wind 2024/25 European winter). The cross-country ranking is unchanged in every year. The roughly ±10% interannual variability of annual wind-and-solar energy is thus a magnitude effect removed by equal-energy normalisation, and its residual effect on the phase metric U — and on the conclusions — is small. This is a supply-side test; the demand shape, set by stable human routines, is held at its single measured year. Reproduction code (analyze_cfe_interannual_robustness.py) is provided with the public repository (see Code Availability).

Supplementary Section S10 — Cross-national supply portfolio optimisation

The domestic portfolio is a non-negative least-squares (NNLS) complementarity problem (Methods (domestic supply portfolio optimisation); Sterl et al. 2020, Tong et al. 2021). Supply is fixed domestic — for demand country c, only the station–technology units of c itself each contribute one basis function — its hourly capacity factor normalised to unit mean — stacked into a matrix Φc. The mixed supply curve sc = Φc wc is fitted to hourly demand through

minwc≥0 Φc wc−dc22​	(S10)

solved by non-negative least squares. After solving, the weights are rescaled to equal annual energy, the residual uncovered share follows the definition of Uc (Methods (domestic mismatch metrics)), and sc is replaced by the mixed curve sc. Because the objective is a squared deviation while the reported metric is the one-sided uncovered energy, we cross-check with a linear program that minimises uncovered energy directly under the same equal-energy constraint. The minimum-uncovered domestic mix solves

minwc≥0, u≥0 t​ut s.t. ut≥dc(t)−(Φcwc)(t),  t​(Φcwc)(t)=t​dc(t)​	(S11)

where ut is the per-hour uncovered slack and t​ut the total uncovered energy. The global minimum-uncovered linear-program benchmark uses a two-step analog — the residual of (S10a), gc(t)=max(dc−Φcwc,0), has its routable part rc(t)=min(φ dc(t),gc(t)) maximally absorbed by an equal-energy non-negative mix of reachable partners’ clean shapes si,

maxw≥0, 0≤cov≤rc t​covt s.t. covt≤i∈Rc(τ)​wi si(t),  t​i​wi si(t)=t​rc(t).​	(S12)

Both are standard linear programs. The NNLS domestic residual is 41.8%, compared with the 40.8% minimum-uncovered linear-program result; the NNLS global residual is 12.7%, compared with the 12.4% minimum-uncovered linear-program floor. The small differences show robustness to objective choice, while only the linear-program values are minima for uncovered energy under the stated equal-energy waveform assumptions. NNLS is retained for the main waveform analysis because its quadratic fit provides stable, distributed basis weights for partner attribution across latency tiers; the metric-aligned linear program serves as the benchmark, and equivalent LP optima can shift weight among alternative bases. Φc contains only the stations of c and aggregates nothing across borders, so the portfolio step involves no cross-border transmission or its efficiency losses; the geographic lever is provided by cross-longitude routing of the compute load (rather than by supply aggregation), as set out in the two-pass procedure and round-trip-time gating below (S11).

Two basis sets are used. Result 1–2 use a domestic station-level basis (461 representative stations assigned to their home country, each country using only its own stations), letting the optimizer select the specific stations most complementary to domestic demand. Result 3 (the AI-workload-shape scenario) uses a country × technology capacity-weighted basis (105 countries × {solar PV, wind} = 210 basis functions) — re-solving the station-level basis over the full 6 operators × multiple years × 6 latitude tolerances × 6 routable fractions × 104 countries (about 4 × 104 NNLS problems of size 8,760 × N) is computationally infeasible. Cross-checking with the station-level basis at the domestic and global corners of τ×φ reproduces the same lever ordering and same-order-of-magnitude costs, so the conclusions of Result 3 do not depend on this basis choice.

Basis crosswalk for Figures 1, 2 and 4. Figure 1 reports 40.1% for the installed-capacity domestic mix. Figure 2 reports 41.8% without routing and 12.7% under fully routable global reach for the station-level NNLS waveform basis and the unperturbed human-traffic profile. Figure 4 uses the country × technology basis for the full AI scenario sweep; its 2030 mixed profile gives 33.3% under domestic reach and 13.9% under fully routable global reach. Because both the demand profile and supply basis differ, the absolute Figure 4 values are not direct alternatives to the Figure 2 values. Figure 4 is interpreted only through AI-shape differences and routing contrasts computed within the same country × technology basis; corner-case station-level re-solves preserve that lever ordering.

The compute load is routed and coupled to the portfolio through a monotonicity-preserving two-pass procedure (Methods (compute-load routing scenarios)) — the domestic portfolio is first solved on the unrouted demand to obtain the residual gap; the routable part of that gap is then, within its network round-trip-time budget (RTT ≤ τ), covered within the same hour by matching its timing to a static non-negative mix of reachable receiving countries’ clean-generation shapes (equal-energy-normalised, magnitude divided out), the covered amount being the elementwise minimum of the two; the domestic portfolio is then re-solved on the post-routing effective demand. The two-pass procedure guarantees that enabling routing never increases the residual.

Supplementary Section S11 — Round-trip-time model and validation against measured Azure latencies

S11.1 Model

Cross-national round-trip time is built from the great-circle distance d(c, i) between capacity-weighted national centroids (Methods (compute-load routing scenarios); Luo & Yang distance model)

L=d(c,i)200 km ms−1+20 ms,  RTT=L×2×1.4​	(S13)

where 200 km/ms is the effective speed of light in fibre (~2/3 c), 20 ms is routing and protocol overhead, the factor 2 is for the round trip and the factor 1.4 for WAN inflation. For the 31-country core sample, measured Microsoft Azure inter-region P50 latencies are used directly (nearest-region mapping); the distance model gives consistent estimates and extends the RTT to all 104 demand countries.

S11.2 Validation

We validate the distance model against measured Azure inter-region P50 latencies (50 regions), covering every country pair mapped to an Azure region (17 mappable countries, 136 pairs). The agreement is strong but the errors are not one-sided: Pearson correlation r (modelled, measured) = 0.865, median absolute error 39 ms and median signed error (modelled − measured) +30 ms. Thus the model overestimates P50 latency on average, with a modelled range of [65, 322] ms compared with a measured range of [11, 336] ms.

Table S19. Modelled and measured Azure round-trip times for representative country pairs.

| Country pair | Modelled RTT (ms) | Measured Azure RTT (ms) | Δ(ms) |
| --- | --- | --- | --- |
| US–GB | 156 | 78 | +78 |
| US–JP | 194 | 164 | +30 |
| US–BR | 155 | 118 | +37 |
| US–IN | 243 | 234 | +8 |
| DE–FR | 66 | 12 | +54 |
| GB–DE | 68 | 17 | +51 |
| JP–KR | 69 | 29 | +40 |
| JP–AU | 151 | 104 | +48 |
| BR–DE | 185 | 195 | −10 |
| AU–US | 263 | 198 | +64 |

The positive bias is largest for short-distance intra-regional pairs carried over Azure's dedicated backbone (for example, DE–FR and GB–DE) and smaller for long-distance intercontinental pairs. The US–IN pair is overestimated by 8 ms, whereas BR–DE is underestimated by 10 ms. The model is therefore positively biased on average but is not a uniformly conservative bound for every routing-relevant pair. The latency scenarios should be interpreted as P50-based model tiers; P95/P99 tail latency and congestion could reduce practical reach.

Supplementary Section S12 — Country sample, routing reach and cloud-region availability

The analysis sample comprises 104 demand countries that simultaneously have a Cloudflare human-traffic curve and station-level renewable supply, distributed across four regional blocks.

Table S20. Country sample by geographic block.

| Geographic block | No. of countries |
| --- | --- |
| Americas | 24 |
| Asia | 30 |
| Europe–Africa–Middle East | 47 |
| Oceania | 3 |
| Total | 104 |

Routing reach is set continuously by the latency tolerance τ through a round-trip-time gate (rather than through nested discrete supply pools) — the receiving countries reachable by sending country c under a latency budget τ are all receivers with RTT(c, r) ≤ τ (RTT model in S11), and this set expands monotonically as τ is relaxed from 50 ms (real-time local only) to 500 ms (fully global reach). The regional blocks (Americas / Europe–Africa–Middle East / Asia / Oceania; see Table S20) are used only to group results for presentation and do not constitute a reachability constraint. Supply is fixed domestically and what is routed is compute load rather than electricity, so HVDC corridors, transmission efficiency and distance ceilings are not involved.

Cloud-region availability imposes a threshold on the routable fraction — a country can receive routed compute only if it hosts at least one large hyperscale cloud provider (AWS / Azure / GCP) region as of 2025; countries without one can act only as sending sources. Within the documented 31-country core, the 20 receivers are AU, BR, CA, CL, DE, FR, GB, ID, IN, IT, JP, KR, MX, MY, NZ, PL, SE, TH, US, ZA (excluded — AR, CO, PE, PY, DZ, GH, LY, MA, NG, TN, UA); across the full 104-country sample about 40 countries meet this condition (i.e. the candidate set for the roughly 39 clean-surplus receiving hubs noted in the main text), spanning North America, Europe, East Asia, the Gulf, Australia–New Zealand, Brazil, Chile, Mexico, India, Southeast Asia, South Africa, Türkiye and Egypt. The complete country-by-country mismatch and grouping table is provided as Supplementary Data 1. 52,53,54

Supplementary Section S13 — Storage cost conversion and duration-dependent storage economics

S13.1 Short-duration conversion

The residual gap is converted into firming storage cost through the NREL battery learning curve (Methods (phase-firming premium)). For a data centre with a 100 MW average load, the annual uncovered energy is Ec = Uc · 8,760 · 100 MWh, and the annualized storage cost is

Cyear=Ecρ⋅capexyear⋅1000⋅(CRF+βOM)365​	(S14)

where the round-trip efficiency is ρ = 0.85, the capital recovery factor is CRF = r(1+r)ⁿ / ((1+r)ⁿ − 1) (discount rate r = 0.07, lifetime n = 15 yr), and the fixed-O&M fraction is βOM = 0.025. Eq. (S14) is only an illustrative fixed-duration lithium-ion conversion; the firming-storage power and energy capacities in the main text and Figure 5, and their cost, are in fact set endogenously by the hourly storage-dispatch optimisation (Methods (phase-firming premium)), which draws on the Li/LDES power costs, energy costs, round-trip efficiencies (Li 0.86, LDES 0.60) and lifetimes given in this section and S13.2, reported as unit cost per megawatt-hour delivered (USD/MWh).

The mid-case lithium-ion overnight capex (USD/kWh) and its three-case range are as follows. 28,29

Table S21. Lithium-ion overnight capex range (USD/kWh).

| Year | Conservative (high) | Central (mid) | Optimistic (low) |
| --- | --- | --- | --- |
| 2025 | 415 | 332 | 236 |
| 2030 | 382 | 289.5 | 194 |
| 2035 | 349 | 247 | 152 |
| 2050 | 333 | 184 | 111 |

S13.2 Duration-dependent technology selection

For the forward-looking costs (Figure 5c,d), the choice and capacity of lithium-ion versus long-duration energy storage (LDES) are set jointly by the hourly storage-dispatch optimisation (Methods (phase-firming premium)), rather than fixed at 4 h lithium-ion or sized by gap percentile. To illustrate why the two cross economically with gap duration, the levelised storage cost (LCOS) of serving a segment of power P and energy E, taking the cheaper technology, is

LCOS=mintech∈{Li, LDES} (CRFtech+FOM) (ptechP+etechE)⋅103Eann ηtech​	(S15)

namely short-duration lithium-ion (power cost p ≈ 280 USD/kW, η = 0.86, 15 yr) and long-duration LDES (p ≈ 1000 USD/kW, η = 0.60, 20 yr, with a much lower energy cost). The per-kWh capacity cost of lithium-ion rises with duration whereas that of LDES is amortized down with duration, the two crossing at ≈ 10 h (consistent with the DOE ≥10 h = long-duration definition); the discount rate is 7 % and FOM is 2.5 %/yr. The 2050 LDES energy cost is 60 / 25 / 12 USD/kWh (conservative / central / optimistic), anchored to Sepulveda et al. (transformational design space $1–20/kWh), the DOE Long Duration Storage Shot (≥10 h systems reaching $0.05/kWh·cycle by 2030), and the public targets of Form Energy and the LDES Council. Because this convention assigns long gaps to long-duration storage, the costs in Figure 5c,d are slightly higher than the fixed 4 h lithium-ion daily-cycling value, and the storage-technology range is wider than the AI-shape range—the uncertainty in forward-looking cost is dominated by the storage-technology pathway.

Supplementary Section S14 — Robustness of supply and routing assumptions

Section S7 quantified the AI-workload parameter uncertainty for Result 3; this section reports the supply-side and routing-side robustness underpinning Results 1–2, all directional single-factor checks (Methods (robustness and sensitivity)).

- (a) The routing model contains no transmission assumptions — the geographic lever moves compute load rather than electricity, riding on the existing internet, with supply fixed domestically (each country still uses its own wind and solar), so it involves no assumptions—HVDC corridor inventories, transmission efficiency or line costs—that would need to be perturbed; the only physical threshold for routing is the network round-trip time (RTT) (see (d)).

- (b) Wind ×1.2 bias correction ∈ [1.0, 1.4] — this changes only the level of the wind capacity factor, not its temporal shape; after equal-energy normalisation the portfolio result is essentially unchanged.

- (c) Representative-station capacity coverage 80 % / 91 % / 96 % — the median uncovered share in the global-reach tier varies by less than 0.5 pp.

- (d) RTT-model parameters (200 km/ms, 20 ms) perturbed by ±20 % — cross-continental RTT estimates vary within about 100–300 ms, mainly affecting whether the 200 ms tier can be achieved across hemispheres; the effect on the uncovered share of the global tier under full routing is below 0.5 pp, and the conclusion that latency benefits saturate at a few hundred milliseconds is unchanged.

- (e) The routable fraction φ is one axis directly swept in the main results — φ ∈ {0, 0.2, 0.4, 0.6, 0.8, 1.0} is given tier by tier as a primary axis in main-text Fig. 2a and Fig. 4d and is not a fixed assumption; its near-linear effect on gap and cost is itself the main result (under global reach, the gap falls from about 35% at 20% routing to 12.7% at full routability).

- (f) Geopolitical feasibility (boundary clause) — this framework imposes no geopolitical, regulatory or data-sovereignty exclusion matrix. These constraints could alter the feasible receiving-country set and routing-basis weights; quantifying their effect requires a dedicated political-feasibility scenario.

Supplementary Section S15 — Boundaries of deliverability and statistical convention

The routing grid in Result 2 computes a phase floor in waveform space. Under equal-energy normalisation it asks whether the routable shortfall belongs to the convex hull of reachable clean-output shapes; absolute surplus is not an input to this geometric problem. Under full routability, the median NNLS residual is 12.7%, while the metric-aligned linear programme gives the exact 12.4% minimum within the same waveform space. Supplementary Section S24 proves the convex-hull and dual characterisation. Physical delivery is a separate quantity-allocation problem requiring absolute hourly budgets.

Shared-budget clearing check. As a due-diligence variant, we impose an illustrative finite hourly clean-surplus budget and re-clear the global-reach, fully routable case. The simultaneously realisable median residual is about 34% under proportional rationing, with allocation-rule and receiver-set variants spanning roughly 28–36%. These values do not revise the 12.7% phase floor because they answer a different question: delivery of finite quantities rather than synthesis of normalised waveforms. They are also not power-system forecasts, because absolute national compute demand, receiving-side GPU capacity, trans-oceanic bandwidth and market clearing remain outside the model.

A separate note on statistical convention — all medians in the main text weight countries equally. Weighting instead by each country’s absolute installed capacity (solar PV and wind MW), the weighted mean of the domestic uncovered share—on the same simple equal-energy-mix basis as the 40.1% headline median—is about 27%; the median and the equal-weighted mean are themselves almost identical (39.7%), so this drop comes entirely from weighting rather than any median-vs-mean effect. Large-capacity countries (such as the United States at 16.8%, and Germany, the United Kingdom and Spain) both have lower mismatch and a large absolute surplus, so the global burden under the energy-weighted convention is smaller than the median country suggests (its policy implication is discussed in the main-text limitations). Reproduction code is provided together with the public repository (see the Code Availability statement).

S15.1 Scope and deliverability ledger for the 12.7% phase floor

The 12.7% phase floor answers a strictly defined geometric question: interactive load, same-hour execution, wind-and-solar supply and static non-negative waveform synthesis. Table S22 separates assumptions that define this waveform space from constraints that belong to physical delivery. Adding firm clean generation, storage or cross-hour deferral expands the waveform space and may lower the floor; adding finite receiver budgets, compute capacity or bandwidth constrains delivery without changing the geometric membership result.

Table S22. Directional ledger for the 12.7% residual (φ = 1, τ = 500 ms, waveform basis).

| Simplifying assumption | Class | Direction on 12.7% | Magnitude / evidence |
| --- | --- | --- | --- |
| Same-hour execution (interactive load cannot defer across hours) | Scope definition | Defines the load class; deferrable training/batch is a different, lower-residual problem | Methods |
| Supply is domestic wind-and-solar only, excluding hydro/nuclear/geothermal firm clean | Scope definition | Isolates wind-solar geometry; adding firm clean would lower the residual but changes the supply question | — |
| Quantity/deliverability placed out of scope (phase only) | Scope definition | Asks about timing alignment, not power delivery | This section |
| Cross-country surplus contention, receiving-side absolute capacity and trans-oceanic bandwidth | In-scope, un-modelled | Raises the realisable residual | Direction is unambiguous (can only raise it); magnitude depends on the allocation scope and capacity assumptions (see analyze_cfe_r2_allocation_sensitivity.py) |
| Demand shaped by request count, not weighted by per-request compute | In-scope, un-modelled | Raises the realisable residual | κ = +1 gives 12.7% → 15.5% (Table S25) |
| Only countries hosting a hyperscale cloud region may receive | In-scope, un-modelled | Raises the realisable residual (fewer receivers) | — |
| Round-trip time via the P50 distance model | Modelling approximation | Mixed approximation; positive median bias but not uniformly conservative | Median signed error +30 ms across 136 Azure P50 pairs; BR–DE is underestimated by 10 ms (Table S19) |
| Tail latency (P95/P99) and session-state migration un-modelled | Modelling approximation | Opposes the row above; not quantified | — |

Conclusion. The 12.7% NNLS result is the reported estimate of the wind-and-solar phase floor, while the metric-aligned LP establishes an exact median floor of 12.4% within the same feasible waveform space. The shared-surplus exercise concerns a different, quantity-delivery estimand and places its illustrative simultaneous residual near one third. Supplementary Section S24 gives the mathematical proof of the phase floor and the conditions under which it can change.

S15.2 Longitude versus waveform shape: empirical anatomy of the phase floor

To identify what sets the global-reach phase floor, we hold the two-pass waveform-matching procedure fixed at the fully routable, 500 ms corner and vary its inputs. Replacing the 40 real hyperscaler partners with synthetic partners spaced uniformly in longitude—each carrying the capacity-weighted mean clean-availability shape, re-phased to its longitude—gives a median floor of 13.0% at 15° spacing and 13.2% at 30°, against 12.7% in the observed dictionary; rolling a high-quality solar partner (Chile) to all longitudes gives 11.5%. Filling the mid-Pacific receiving void therefore does not remove the floor. The binding geometry is the span of the available waveforms rather than the absence of a receiving longitude.

Holding longitude uniform, we then vary the clean-supply dictionary. A purely solar dictionary gives a 10.7% floor, the observed solar-plus-wind dictionary 13.0%, and a wind-heavier dictionary 15.1%. About 50% of the uncovered residual falls in the local 18:00–23:00 evening window and only 14% in the 00:00–05:00 deep night. These values are empirical evaluations of different convex hulls, not substitutes for the mathematical proof: longitude changes leave the feasible waveform space nearly unchanged, whereas a new supply dictionary changes that space and therefore changes its distance from demand. Reproduction code (analyze_cfe_r2_longitude_counterfactual.py) is provided with the public repository (see Code Availability).

Supplementary Section S16 — Robustness of full electricity-supply cost to reliability target, annual cycling and storage-cost scenario

The full electricity-supply cost in the main text (domestic generation, firming storage and ancillary services, excluding transmission) is determined by hourly storage-dispatch optimisation — for each country a linear program co-optimises the generation overbuild and the power and energy capacities of lithium-ion (Li) and long-duration energy storage (LDES) and their hourly charge/discharge, satisfying the hourly energy balance under round-trip efficiency, state-of-charge (SOC) recursion, annual cycling and curtailment, with unserved load priced at the value of lost load (VOLL) (Methods (phase-firming premium)). Firming-storage capacity is endogenous to this optimisation, so percentile sizing and associated P90–P100 sensitivity are not part of the model. This section tests the robustness of the cost to three key choices of the dispatch model.

Reliability target and annual cycling. Under the 2030 mixed-AI shape, relaxing the energy-reliability target from 100% to 99.9% and 99%, and toggling the annual SOC-cycling constraint, leaves the full electricity-supply cost of representative countries essentially unchanged (Table S23, differences < 1%) — at the standard value VOLL = 10,000 USD/MWh, building extra storage to cover the tail gaps remains cheaper than shedding load, so the model serves ≈100% throughout the 99%–100% range; nor is the cost driven by cross-seasonal storage. The headline therefore does not depend on the precise reliability threshold or cycling constraint.

Table S23. Robustness of the full electricity-supply cost to the reliability target and annual SOC cycling (representative countries, 2030 local tier, USD/MWh).

| Country | 100% · cyclic | 99.9% · cyclic | 99% · cyclic | 99.9% · non-cyclic |
| --- | --- | --- | --- | --- |
| United States | 112 | 112 | 112 | 112 |
| Germany | 142 | 142 | 142 | 142 |
| Palestine | 483 | 483 | 483 | 483 |

Storage-cost scenario. The year-by-year storage capex follows conservative / central / optimistic scenarios (S13, spanning roughly ±50% in Li and LDES energy cost); storage is the component that drives the cost’s variation with reach, so the cost scenario is the dominant uncertainty band, yet the cost advantage of the global tier over the local tier holds consistently across the three scenarios. Reproduction code is provided together with the public repository (see the Code Availability statement).

Supplementary Section S17 — The demand-shape model, compute-intensity weighting, and robustness to the flat-baseload share

S17.1 Scope

The demand curves in this study are not raw Cloudflare traffic but the output of an explicit demand-shape model. Its input is the country-level hourly rate of human web requests on Cloudflare Radar; the model maps this request stream to the intraday shape of user-facing, network-latency-constrained interactive/inference electricity use that can be served from a remote location, by weighting each request class by its mean per-request energy (Methods). Under the model's assumption that the intraday class composition is approximately stationary the request rate fixes this shape, and after equal-energy normalisation only the shape enters the analysis; we do not treat raw traffic as electricity. The stationary-composition assumption and the resulting shape are corroborated against real inference-serving traces (S17.2) and stress-tested by the flat-baseload share (S17.3) and compute-intensity weighting (S17.4). Flat training and batch baseload (model training, cache and CDN hits) have no diurnal rhythm and is carried separately by the flat training share λtrain in the AI workload operator (Methods (AI workload perturbation)), so it does not enter the diurnal shape. 9

S17.2 Real-trace corroboration of the shape

The diurnal shape (evening peak) of the human-traffic proxy agrees independently with two real large-language-model inference service loads — BurstGPT v2 (Supplementary Section S2) and the Azure LLM Inference Trace 2024 (44 million requests, Supplementary Section S4); the diurnal power of both, converted from request rate, exhibits an evening peak. For the interactive/inference service tier that is of true interest here, the phase of the proxy is therefore corroborated by real service loads rather than assumed.

S17.3 Robustness to the flat-baseload share

To delineate what the headline figures would be if real data centre load were flatter than human traffic, we mix each country’s demand with an entropy-preserving flat component, dβ=(1−β) dhuman+β ⟨dhuman⟩, where β is the flat-baseload share, and recompute the Result 1 country-level equal-energy mismatch and the global routing residual (φ = 1, τ = 500 ms, waveform). Table S24 reports the median across 104 countries.

Key point — flattening the load does not reduce the mismatch but slightly raises it (Result 1 rises from 40.1% to 41.5%, and routing residual falls marginally from 12.7% to 12.0%). The reason is that the mismatch is constrained by the roughly half-day window in which solar PV output is zero at night, not by the shape of the evening demand peak; flattening the load merely shifts demand from the evening peak to the equally sunless deep night, leaving the total uncovered share almost unchanged. The headline figures are therefore robust to whether real data centre load is flatter than human traffic and conservative with respect to admixing flat baseload.

Table S24. Robustness of the headline metrics to the flat-baseload share β (median across 104 countries, %).

| Flat-baseload share β | Result 1 country mismatch | Global routing residual (φ=1, τ=500) |
| --- | --- | --- |
| 0 (pure human traffic) | 40.1 | 12.7 |
| 0.25 | 40.8 | 12.1 |
| 0.50 | 41.5 | 12.0 |

S17.4 Robustness to compute-intensity weighting

The request-to-electricity mapping above assumes that the diurnal composition of request types is broadly stationary. If that assumption were broken—if high-compute-intensity requests (large-language-model conversation, code, and image/video generation) were more concentrated in the interactive evening peak than low-compute-intensity requests (search, static pages, and CDN hits)—then reweighting by per-request energy would sharpen the evening demand peak; the opposite composition would flatten it. We characterise this directly with a single compute-intensity concentration exponent κ — modelling per-request energy as e(t)∝(R(t)/⟨R⟩)κ, the energy-weighted demand shape is dκ(t)∝R(t)1+κ (rescaled to each country’s original annual total). κ=0 is the request-count headline; κ>0 concentrates high-compute requests at the evening peak (sharper); κ<0 is low-compute/CDN-dominated (flatter). For each κ we recompute the national equal-energy mismatch (R1) and the global routing residual (R2; φ=1, τ=500 ms, waveform match); Table S25 gives the 104-country median.

The national mismatch is robust to compute intensity—R1 stays within 40.1–41.5% across κ∈[−0.5,+1.0]—while the routing residual rises monotonically with compute intensity (12.0% → 12.7% → 15.5%), because pushing demand further into the solar-absent evening only lowers phase-alignability. Hence if the true load is more compute-heavy than the raw request count, the headline mismatch is essentially unchanged and the routing residual can only be higher; the main-text figure (12.7%) is therefore on the conservative (low) side of the residual, and the conclusion is robust—indeed conservative—to building the demand shape from request counts rather than from compute.

Table S25. Robustness of the headline metrics to the compute-intensity concentration exponent κ (104-country median, %).

| Compute intensity κ | R1 national mismatch | Routing residual (φ=1, τ=500) |
| --- | --- | --- |
| −0.50 (low-compute / flatter) | 41.2 | 12.0 |
| −0.25 | 40.6 | 12.2 |
| 0.00 (request count, headline) | 40.1 | 12.7 |
| +0.50 | 40.4 | 14.0 |
| +1.00 (high-compute / sharper) | 41.5 | 15.5 |

Reproduction code (analyze_cfe_s17_computeintensity.py) is provided together with the public repository (see the Code Availability statement).

Supplementary Section S18 — Scenario anchoring of the latency tolerance τ — measured service-level targets of real workloads and the compute-to-network-budget conversion

S18.1 The conversion

The main text and Sections S11 and S12 define the latency tolerance τ as the network round-trip time a single request can tolerate, which gates the reachable receiving countries (τ = 50 ms local only; 100–150 ms reaches about 3,000–6,700 km; 200–300 ms near-global; ≥ 500 ms global). Which tier a real workload falls in is not set directly by its end-to-end service-level target (SLA), but by the round-trip budget a single request actually leaves for the network once on-server compute/generation is subtracted

network budget=end-to-end SLA−on-server compute time.​	(S16)

The conversion places workloads along a broad continuum of network budgets. Voice assistants, inline code completion and real-time retrieval-augmented generation have sub-second service targets, and their on-server compute can leave only tens of milliseconds for the network; under the assignments used here they generally remain in local or short-reach tiers. Image and video generation and batch inference take seconds to hours per request, so trans-oceanic round-trip time is small relative to their execution time and they are assigned broader reach. These are modelled workload tiers rather than hard feasibility classes: actual routing also depends on tail latency, session state, data governance and available compute capacity. Table S26 reports the published service targets, measured compute times and the reasoned conversion to each τ tier.

Table S26. Measured service-level targets, compute times and assigned latency-tolerance tiers of real workloads.

| Workload scenario | Measured end-to-end SLA | Compute/generation time | Network round-trip budget | τ tier | Source |
| --- | --- | --- | --- | --- | --- |
| Inline code completion (Google internal IDE) | < 100 ms end-to-end | ~40 ms model serving | ~60 ms | 50 ms local | 65 |
| Code completion (GitHub Copilot production) | < 200 ms mean | not itemized | < 200 ms (unsplit) | 50 ms local | 64 |
| Real-time voice assistant (measured pipeline) | < 1 s (acceptable threshold) | 0.94 s (ASR 0.05 + LLM TTFT 0.11 + generation 0.67 + TTS 0.29) | ~60 ms | 50 ms local | 67,68,69 |
| Real-time retrieval-augmented generation (low-latency schedule) | TTFT ~0.03 s | retrieval > 80% | ~0 | 50 ms local | 63 |
| Interactive chat (tight SLO) | TTFT 0.25 s / TPOT 0.1 s | prefill ~0.1 s | ~150 ms | 100–150 ms regional | 60,61 |
| Web search (tolerated added delay) | +200–400 ms → −0.29 to −0.59% daily searches | not a compute term | ~200–400 ms (engagement cost) | 100–150 ms regional | 70,71 |
| Interactive chat (loose QoE target) | TTFT target 1.3 s | sub-second prefill | ~1 s | 200–300 ms near-global | 62 |
| Throughput-optimized retrieval-augmented generation | TTFT ~2.47 s | ~1.54 s | ~0.9 s | 200–300 ms near-global | 63 |
| Server-side image generation (accelerated diffusion, H100, 1024²) | tolerance ~1 s | 0.1–1.1 s | ~0.9 s to unbounded | 200–300 ms near-global | 72,71 |
| On-device / slow image generation | tolerance ~10 s | up to 12 s | unbounded | 500 ms+ cross-hemisphere global reach | 73,71 |
| Video generation | no interactive SLA | 32 s–~30 min | unbounded | 500 ms+ cross-hemisphere global reach | 74,75 |
| Batch inference (up to 55% of serving capacity is offline) | 24 h | seconds, negligible | unbounded | 500 ms+ cross-hemisphere global reach | 76,77,78 |

Measured inter-continental round-trip times as a reachability reference — New York–London about 59–80 ms; Los Angeles–Tokyo about 100–115 ms; United States–Australia about 140–200 ms; Tokyo–London about 226 ms; the global antipodal ceiling is about 340 ms (a long-haul-link threshold of 57 ms per 5,700 km at 2/3 c propagation, ; modelled values in S11, Table S19). 79

S18.2 Strength and reasoning notes

Most cells of Table S26 are given directly by public sources, but the following are reasoned rather than directly quoted and are flagged conservatively — (i) the ~0.1 s prefill compute for the interactive-chat tight SLO tier is derived by dividing 512 tokens by the ~5,000 tokens/s prefill throughput (the two figures come separately from DistServe and Andes), and the prefill throughput is a hardware-specific measured value, not a universal constant; (ii) GitHub Copilot reports only a < 200 ms mean end-to-end figure without splitting compute from network, so its local-tier assignment is reasoned; (iii) the 200–400 ms for web search is a tolerated added delay / engagement cost, not a compute time or a hard SLA, so mapping it to the regional tier is an interpretation; (iv) the unbounded network budgets for image/video generation and batch inference are qualitative judgements (per-request compute far exceeds the ~340 ms inter-continental round-trip ceiling), not exact subtractions; (v) every τ-tier assignment applies the reach thresholds of the main text and Sections S11–S12 (50 / 100–150 / 200–300 / ≥ 500 ms) rather than re-deriving them from the cited latency sources. These flags do not change the directional two-ended conclusion. 60,62

Supplementary Section S19 — Positioning relative to prior work

The table below compares this study with the closest prior work along six dimensions. This study combines globally resolved human digital activity in local phase, station-level wind and solar output and measured network latency, and quantifies the wind-and-solar phase floor of same-hour cross-longitude migration.

Table S27. Dimensional comparison of this study with the closest prior work.

| Study | Actual network-latency gating | Global country coverage (104 countries) | Local-phase human demand curve | Station-level wind/solar supply | Routable-fraction axis | Phase-alignability / geometric residual |
| --- | --- | --- | --- | --- | --- | --- |
| Carbon-aware temporal scheduling 5,6 | — | — | — | — | — (time axis only) | — |
| Single-operator spatiotemporal migration 7 | — | — | — | partial | — | — |
| Latency–geography framework (Luo & Yang) 8 | stylized (RTT distance) | — | — | — | partial | — |
| Global wind–solar interconnection (moves electricity, not compute) 15 | not applicable | ✓ | — | partial | — | — |
| This study | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |

Supplementary Section S20 — Independent external corroboration (E3)

Two findings of an independent industry analysis of data-centre load growth (E3) support, from outside our framework, the two pillars of this study — the diurnal shape of interactive demand, and the firm-capacity residual that renewables leave behind. 59

S20.1 Diurnal-shape corroboration

E3 independently anticipates the demand-shape premise of our study. Its analysis notes that data centres have historically been high-load-factor, near-flat facilities dominated by baseload and training, but that as user-facing utilisation (inference) overtakes training, the daily load curve comes to be shaped by human use — a business-oriented tool peaks in working hours, whereas a personal tool develops evening peaks resembling residential load. Figure S5 places E3’s modelled scenarios alongside our empirical interactive curve. The direction agrees — E3’s personal-use scenario peaks in the evening, coincident with our interactive-demand peak — while the amplitude contrast makes the case for isolating the interactive slice — E3’s total-load curve stays within about ±5% of flat because baseload dilutes the signal, whereas our interactive-only curve (the Cloudflare median) swings from about 0.3 to 1.4 times its daily mean. The comparison is qualitative — E3’s daily curves are modelled illustrations of total U.S. data-centre load, not measured data, so they corroborate the direction of our premise and the rationale for the flat-baseload separation (S17.3) rather than validating our hourly curve, for which the empirical anchors are the BurstGPT and Azure traces (S2, S4). 59

Figure S5 | Independent qualitative corroboration of the diurnal-shape premise (E3). E3’s modelled daily shapes of total U.S. data-centre load (digitised from Figure 5 of E3; each curve integrates to one over 24 h) are compared with our empirical 104-country median interactive-demand share by local hour, all on a daily-mean = 1 basis. (a) E3’s two usage scenarios — a business tool peaks in the afternoon (about 14 h), a personal tool in the evening (about 19 h, coincident with our interactive peak at 20 h, dashed line); the y-axis is zoomed because total-load variation is small. (b) Amplitude contrast on a common axis — E3’s total-load curve stays within about ±5% of flat because baseload and training dilute the interactive signal, whereas our interactive slice alone swings from about 0.3 to 1.4 times the daily mean. The comparison is directional (E3’s curves are modelled illustrations of total load, not measured hourly data), supporting the shape premise and the flat-baseload separation, not reproducing our curve. 59

S20.2 The firming residual

An independent grid-planning assessment of U.S. data-centre load growth (E3) reaches the same conclusion as our full-system cost result (Result 3) from a different starting point and with a different method. Using an effective-load-carrying-capability (ELCC) framework, that study finds that meeting 100% of the incremental data-centre energy demand with renewables would require about 115 GW of nameplate wind and solar by 2030, yet these deliver only about 23 GW of effective (reliability) capacity — roughly one-fifth of nameplate — leaving a residual firm-capacity gap of about 16 GW that must still be met by storage or other firm resources; the effective contribution of solar paired with short-duration storage itself declines over the same horizon (its ELCC falling from about 0.5 in 2024 to about 0.35 in 2030). 59

The implication is the same as ours — building enough renewables to supply all of the energy does not close the timing gap. A substantial firming residual persists because renewable output does not arrive at the hours of peak need — the mismatch is temporal, not volumetric. This is precisely the mechanism behind our finding that cross-longitude routing is cheaper almost entirely through the firming storage it avoids rather than through cheaper generation, and behind the paper’s central claim that capacity expansion inside a single grid cannot close a phase-set mismatch. Here that claim is corroborated by an independent, U.S. grid-planning ELCC calculation rather than by our hourly-alignment framework.

The comparison is directional rather than a reproduction of our figures — E3 is U.S.-specific and reports a capacity credit (reliability contribution at peak) rather than our hourly uncovered-energy share, and it addresses total data-centre load rather than the interactive slice. It therefore supports the mechanism and the direction of our cost result, not our specific residual percentages.

Supplementary Section S21 — Identities linking the wind-and-solar matching gap U to established metrics

The main text uses the hourly wind-and-solar matching gap Uc defined in Eq. (1) as its matching metric. This section proves that, under equal-energy normalisation, Uc is mathematically identical to four established metrics drawn respectively from corporate procurement, distributed energy, net-zero-energy buildings and statistics, so the matching-gap result reported here can be cross-checked against those established conventions rather than being a bespoke measure.

Let a country’s hourly demand d(t)≥0 and clean supply s(t)≥0 each be normalised to the same annual total t​d=t​s=E, and write U=t​max(d−s,0)/E.

Proposition 1 (self-sufficiency). U=1−SS, where the temporal self-sufficiency rate SS=t​min(d,s)/t​d (Luthander et al. 2015). Proof. Pointwise d=min(d,s)+max(d−s,0); summing over t and dividing by E gives 1=t​min(d,s)/E+U. This step needs only t​d=E. ∎ 18

Proposition 2 (24/7 CFE matching score). 1−U equals, hour by hour, the 24/7 carbon-free energy matching score CFE=t​min(d,s)/t​d (Google 2020; Riepin & Brown 2024). Proof. Google’s convention caps single-hour carbon-free supply at that hour’s demand and does not carry surplus forward — mathematically min(d,s); by Proposition 1, CFE=t​min(d,s)/E=1−U. ∎ 3,4

Proposition 3 (load match index). 1−U equals the hourly load match index of net-zero-energy buildings, fload=t​min(d,s)/t​d (Sartori et al. 2012). With the annual energy balanced (their net-zero line, i.e. our t​d=t​s), that literature reads the hourly load match index as pure day–night phase mismatch, isomorphic to our convention. 19

Proposition 4 (total variation / overlap). Let p=d/E, q=s/E be two discrete probability distributions. Then U=12t​|d−s|/E=TV(p,q) and 1−U=t​min(p,q)=OVL(p,q), so U is the total-variation distance between the two normalised shape distributions and 1−U their overlap coefficient (Weitzman 1970). Proof. Write Md=t​max(d−s,0), Ms=t​max(s−d,0). From t​d=t​s=E, Md=Ms; since |d−s|=max(d−s,0)+max(s−d,0), 12t​|d−s|=12(Md+Ms)=Md=UE. ∎ 20

Equal energy is indispensable. Propositions 2 and 4 rely on Md=Ms (i.e. ∑d=∑s). Dropping equal-energy normalisation gives a counterexample — for d=(2,0), s=(0,0), U=∑max(d−s,0)/∑d=2/2=1, whereas 12∑|d−s|/∑d=(2+0)/(2⋅2)=0.5≠U. This shows that our equal-energy normalisation is precisely what removes the volume shortfall dimension so that U measures phase mismatch alone; the un-normalised real CFE score also carries a structural energy shortfall.

Under equal-energy normalisation, U=1−SS=1−CFE=1−fload=TV=1−OVL forms a closed chain of equivalences (Table S28). The original definition of Uc is in S8, its optimisation forms (NNLS and L1-LP) in S10, and the phase-alignability context of the 12.7% residual in S15.

Table S28. Metric definitions and equivalences.

| Metric | Definition | Relation to U | Source |
| --- | --- | --- | --- |
| 24/7 CFE matching score | Σ min(d,s)/Σ d (hourly-capped) | 1−U | Google 2020; Riepin & Brown 2024 3,4 |
| Temporal self-sufficiency SS | Σ min(d,s)/Σ d | 1−U | Luthander et al. 2015 18 |
| Load match index f_load | Σ min(d,s)/Σ d (annual-balanced) | 1−U | Sartori et al. 2012 19 |
| Overlap coefficient OVL | Σ min(p,q) | 1−U | Weitzman 1970 20 |
| Total-variation distance TV | 12t​¦p−q¦ | U | — |

The script reproducing these identities and the counterexample is provided with the public repository (see Code Availability).

Supplementary Section S22 — Converting the phase-firming premium Π to absolute levelised cost

The cost side of the main text uses the dimensionless phase-firming premium  defined in Eq. (7) as its metric. This section gives the year-by-year conversion between  and its absolute basis — the full electricity-supply cost per MWh of digital demand (generation + firming storage + ancillary services) — so readers can recover the absolute figures and cross-check them. The absolute cost is determined by hourly storage-dispatch optimisation (firming-storage power and energy capacities set endogenously, not percentile-sized; see S13 and Methods (phase-firming premium)), with anchors — 2030 global firm ≈ 184, on-site ≈ 220; storage component ≈ 60 / 77 USD/MWh. Table S29 gives the 104-country year-by-year medians under the expected mixed-AI shape for the on-site (τ = 50 ms, φ = 0) and global-reach (τ = 500 ms, φ = 1) tiers (USD/MWh; Π and σ in per cent).

Table S29. Year-by-year conversion between the phase-firming premium Π and absolute levelised cost (expected mixed-AI shape; hourly storage-dispatch basis).

| Tier | Year | Generation | Firming storage | Ancillary | Firm total | Π | σ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| On-site (τ = 50 ms, φ = 0) | 2025 | 170 | 91 | 6 | 265 | 53% | 35% |
|  | 2030 | 135 | 77 | 6 | 220 | 61% | 38% |
|  | 2035 | 122 | 73 | 6 | 202 | 63% | 39% |
|  | 2040 | 111 | 68 | 6 | 184 | 66% | 40% |
|  | 2045 | 102 | 62 | 6 | 170 | 66% | 40% |
|  | 2050 | 92 | 56 | 6 | 156 | 68% | 40% |
| Global reach (τ = 500 ms, φ = 1) | 2025 | 144 | 68 | 6 | 220 | 47% | 32% |
|  | 2030 | 119 | 60 | 5 | 184 | 55% | 36% |
|  | 2035 | 107 | 56 | 5 | 168 | 57% | 36% |
|  | 2040 | 96 | 52 | 5 | 152 | 59% | 37% |
|  | 2045 | 89 | 48 | 5 | 141 | 59% | 37% |
|  | 2050 | 82 | 44 | 5 | 129 | 62% | 38% |

Trend direction — absolute cost falls, relative premium rises. Table S29 exposes a fact revealed by normalisation and hidden by the absolute convention — the firm full-system cost falls year by year as generation and storage cheapen (global 220→129, on-site 265→156), yet the phase-firming premium Π rises year by year (global 47%→62%, on-site 53%→68%). The mechanism is in the denominator — the wind-and-solar generation built (with firming overbuild) falls faster over 2025–2050 (global 144→82 USD/MWh), whereas the firming storage that fills the phase mismatch declines far more slowly (global 68→44), so the weight of storage relative to generation — i.e. Π — rises rather than falls. In other words, as clean generation grows ever cheaper, this mismatch tax takes an ever-larger share of the clean electricity price, consistent with the main text’s finding that the phase constraint does not recede with technological progress but grows in relative economic weight. σ=Π/(1+Π) gives the equivalent reading on a common [0,1) scale. Year-by-year Π is the country-level median of (firm−gen)/gen, consistent with Fig. 5c. Firming-storage capacity is set endogenously by the hourly dispatch optimisation (not percentile sizing), so there is no P90–P100 sizing-percentile sensitivity; the cost is robust to the reliability target (99%–100%) and to annual SOC cycling.

The script reproducing this table is provided with the public repository (see Code Availability).

Supplementary Section S23 — Literature basis and derivation of the two dimensionless measures

This section reviews the design motivation, definitions, derivation and literature lineage of the pair of dimensionless metrics carried throughout the paper—the hourly wind-and-solar matching gap U and the phase-firming premium Π—showing that they are not devised by this study but inherit established conventions from corporate procurement, energy systems, buildings and statistics. The formal proofs of the identities between U and established metrics, together with the de-normalisation counterexample, are given in S21 (Table S28); the year-by-year conversion between Π and absolute levelised cost is given in S22 (Table S29). This section does not repeat those derivations but sets out the sources and the overall logic.

Design motivation and shared premise. The ideal measure would account, country by country, for the absolute electricity use of digital load and its carbon-free supply gap. But user-facing interactive and inference load is spread across the globe; its true draw is not metered country by country and hour by hour, and the absolute energy of compute can only be triangulated within wide bounds (see S1), so the complete absolute demand curve cannot be obtained at global scale. What can be robustly observed is waveform rather than magnitude — the intraday rhythm of human digital activity is modelled from Cloudflare network request traffic (see S17), and that of clean output by station-level capacity factors (see S9). We therefore normalise each country’s demand dc(t) and supply sc(t) curves to the same annual total t​dc=t​sc=Ec, stripping out the unobtainable absolute magnitude and retaining only the phase relation of the two waveforms. This normalisation turns the intrinsic data limitation into a methodological demarcation — every metric measures only temporal phase mismatch and admits no structural energy shortfall; the counterexample showing this premise cannot be dropped is given in S21.

Definition and lineage of the wind-and-solar matching gap U. The hourly wind-and-solar matching gap is defined as the share of demand exceeding contemporaneous supply under equal-energy normalisation,

Uc=t​maxdc(t)−sc(t), 0t​dc(t)∈[0,1].	(S17)

Its authoritative anchor is the 24/7 carbon-free energy (CFE) standard — its complement 1−Uc equals hour by hour the 24/7 CFE matching score—the share of annual demand met by carbon-free supply capped at each hour’s demand with surplus not carried forward. This standard was introduced by Google’s 24/7 CFE initiative (Google 2020) and established through system-level assessment (Riepin & Brown 2024), and is now in wide use across corporate clean-power procurement and policy. Under the equal-energy premise, the same 1−Uc also equals hour by hour the temporal self-sufficiency rate in the distributed-energy literature (Luthander et al. 2015) and the load match index of net-zero-energy buildings (Sartori et al. 2012), while Uc itself equals the total-variation distance between the two normalised shape distributions and its complement the overlap coefficient in statistics (Weitzman 1970). The four identities form a closed chain U=1−SS=1−CFE=1−fload=TV=1−OVL, letting the same value be cross-checked by these four established conventions; the term-by-term proofs of Propositions 1–4, the closed chain and the de-normalisation counterexample are given in S21. 3,4,18,19,20

Definition and lineage of the phase-firming premium Π. To quantify the firming cost of the same residual phase mismatch, the phase-firming premium is defined as the relative increment, over pure generation cost, of firming the residual phase gap with domestic storage,

Πc=LCOEcfirm−LCOEcgenLCOEcgen∈[0,∞),	(S18)

where the firm full-system cost per MWh comprises generation, firming storage and ancillary services (construction in S13, S16). This definition is the standard practice of the integration-cost (system-LCOE) framework for variable renewables — Ueckerdt et al. (2013) introduced system LCOE to add the integration cost of variable renewables on top of bare generation cost; Hirth et al. (2015) provided the economic framework for integration costs; and Joskow (2011) showed that comparing intermittent with dispatchable sources by bare levelised cost alone systematically understates the former—precisely why this paper reports the cost with Π rather than bare LCOE. Πc is dimensionless and zeroed at perfect phase alignment (the generation floor, Π=0); to lie on the same [0,1) scale as Uc it can be written equivalently as the phase-chasing cost share σc=Πc/(1+Πc). The premium implied here falls within the range reported by IRENA (2026) for the reliable 24/7 delivery of comparable wind and solar plus storage; the year-by-year conversion of Π to absolute USD levelised cost, and the reversal whereby absolute cost falls while the relative premium rises, are given in S22. 11,12,35,36

Relationship between the two. U and Π are two readings of the same physical quantity—the phase mismatch between demand and clean supply at the hourly scale. The better the phase alignment, the lower U, the fewer continuous gap segments firming storage must fill, and the closer Π falls to zero; the phase-unalignable residual turns, through firming storage and ancillary services, into the premium. The paper therefore takes no absolute figure of energy, capacity or money as its endpoint, working throughout on this pair of observable, cross-checkable metrics rooted in the established literature.

Table S30. Concept–reference map for the two dimensionless temporal-mismatch measures.

| Concept | Relation to metric | Source (main-text reference number) |
| --- | --- | --- |
| 24/7 CFE matching score | =1−U | Google 2020; Riepin & Brown 2024 3,10 |
| Temporal self-sufficiency rate (PV self-consumption) | =1−U | Luthander et al. 2015 27 |
| Net-zero-energy-building load match index | =1−U | Sartori et al. 2012 28 |
| Total-variation distance / overlap coefficient | U=TV, 1−U=OVL | Weitzman 1970 29 |
| System LCOE / integration cost | Π=integration cost / generation cost | Ueckerdt et al. 2013; Hirth et al. 2015 8,9 |
| Bare LCOE understates intermittent sources | Rationale for Π | Joskow 2011 44 |
| Economics of reliable wind-and-solar delivery | Established calibration of Π’s magnitude | IRENA 2026 45 |

Supplementary Section S24 — Mathematical characterization and proof of the wind-and-solar phase floor

This section establishes the phase floor as a property of a precisely defined waveform-synthesis problem. The proof has four parts: the uncovered share is a total-variation distance under equal-energy normalisation; static non-negative mixing generates a convex hull of reachable clean-output waveforms; the metric-aligned linear programme and its dual give an exact separation certificate; and changes in demand or the supply dictionary obey explicit monotonicity and stability relations.

S24.1 Equal-energy identity

Let p and q be non-negative hourly demand and supply waveforms normalised so that their elements each sum to one. The uncovered share, total-variation distance and one minus the hourly overlap are identical:

U(p,q)=tptqt+=12pq1=1−tmin{pt,qt}.	(S19)

Proof. Write at = [pt − qt]+ and bt = [qt − pt]+. Equal total energy implies Σt at = Σt bt. Hence ‖p − q‖1 = Σt(at + bt) = 2Σt at, proving the first two equalities in (S19). Moreover, min{pt,qt} = pt − at; summing over t and using Σt pt = 1 proves the overlap identity. Thus the paper's uncovered-share metric is exactly a distance between temporal probability distributions, not a proxy based on scale.

S24.2 Feasible waveform space and zero-floor condition

For country c, normalise the routable shortfall to r̄c and every reachable receiving profile to s̄i. Because each profile has unit total energy, the equal-energy constraint turns any static non-negative mixture into weights that sum to one. The feasible set is therefore exactly the convex hull of the reachable profiles, and the phase floor is its total-variation distance from the shortfall:

Cc(τ)=conv{si:i∈Rc(τ)},ρc(τ)=minq∈Cc(τ)12rcq1.	(S20)

Proposition 1. The phase floor is zero if and only if r̄c belongs to Cc(τ). Proof. If r̄c belongs to the convex hull, a feasible set of weights reproduces it and the norm in (S20) is zero. Conversely, a zero norm implies equality between r̄c and a feasible convex combination, so r̄c belongs to the convex hull. In a finite hourly space the convex hull of finitely many profiles is closed, so a demand waveform outside it has strictly positive distance.

S24.3 LP equivalence and dual separation certificate

The overlap-maximising LP in (S12) is the primal representation of (S20). By (S19), maximising covered overlap is equivalent to minimising total variation. The corresponding dual is

ρc(τ)=max0≤h≤1[hTrc−maxi∈Rc(τ)hTsi].	(S21)

Proposition 2. Equations (S20) and (S21) have the same optimum. Proof. For probability vectors, TV(p,q) = max0≤h≤1 hT(p − q). Substituting q from the convex hull gives a minimisation over q and a maximisation over the compact cube 0≤h≤1. Both sets are compact and convex and the objective is bilinear, so the minimisation and maximisation may be interchanged. Minimising −hTq over the convex hull selects the profile with largest hTs̄i, yielding (S21). A positive optimum therefore supplies an explicit separating hour-weight vector h: the weighted mass of the demand shortfall exceeds that of every feasible clean-output waveform.

Numerical certificate. The metric-aligned LP was solved on the same 8,760-hour basis used for Figure 2. Under fully routable global reach its optimum is positive for every one of the 104 countries: the minimum is 5.54%, the first quartile 10.95%, the median 12.37%, the third quartile 14.07% and the maximum 22.04%. The NNLS-minus-LP difference has a median of 0.17 percentage points, a 90th percentile of 0.50 and a maximum of 0.95. Thus the 12.7% NNLS result is a close, attribution-stable estimate of a strictly positive 12.4% metric-aligned floor, rather than an artefact of the squared-error objective.

S24.4 Waveform synthesis versus finite delivery

The profiles s̄i in (S20) are dimensionless basis functions. Reusing one profile in different countries' approximation problems repeats a function, not a unit of energy, and therefore creates no conservation violation inside the waveform estimand. A physical delivery problem instead requires variables xcit with energy units and an hourly receiver budget:

Fdel={x:cxcit≤Bit∀i,t}.	(S22)

Equation (S22) cannot be identified from equal-energy waveforms alone because the absolute budgets Bit and country-level compute demand are not observed. The finite shared-surplus analysis in S15 assigns illustrative budgets and therefore answers a separate deliverability question. Its 28–36% range neither replaces nor disproves the 12.7% phase floor; the two quantities refer respectively to finite energy allocation and dimensionless waveform synthesis.

S24.5 Dependence on the dictionary and demand waveform

The word floor is relative to the specified feasible waveform space. Two general relations make that scope explicit:

C1⊆C2⇒ρ(d,C2)≤ρ(d,C1),|ρ(d1,C)−ρ(d2,C)|≤TV(d1,d2).	(S23)

Proof. The first relation follows because minimising distance over a superset cannot give a larger minimum. For the second, the triangle inequality gives ρ(d1,C) ≤ TV(d1,d2) + ρ(d2,C); exchanging d1 and d2 yields the absolute-value bound. Hence storage, firm generation or new supply profiles can lower the floor by expanding C, while a change in demand can move it by no more than the total-variation size of that change.

The counterfactuals follow these relations. Uniform longitude placement gives 13.0–13.2%, showing that it adds little effective waveform span beyond the observed global dictionary. Replacing the dictionary changes the convex hull and moves the floor from 10.7% for solar only to 15.1% for a wind-heavier set. Compute-intensity weighting changes the demand vector and moves the result from 12.0% to 15.5%. These experiments locate the theorem's inputs; the strictly positive LP optimum and its dual, rather than the sensitivity ranges, establish the phase floor.

Supplementary References

1. International Energy Agency. Energy and AI (IEA, 2025). https://www.iea.org/reports/energy-and-ai/executive-summary

2. International Energy Agency. Key Questions on Energy and AI (IEA, 2026). https://www.iea.org/reports/key-questions-on-energy-and-ai

3. Google. 24/7 by 2030: Realizing a Carbon-free Future (Google, 2020). https://www.gstatic.com/gumdrop/sustainability/247-carbon-free-energy.pdf

4. Riepin, I. & Brown, T. On the means, costs, and system-level impacts of 24/7 carbon-free energy procurement. Energy Strategy Reviews 54, 101488 (2024). https://doi.org/10.1016/j.esr.2024.101488

5. Radovanović, A. et al. Carbon-aware computing for datacenters. IEEE Transactions on Power Systems 38, 1270–1280 (2023). https://doi.org/10.1109/TPWRS.2022.3173250

6. Wiesner, P., Behnke, I., Scheinert, D., Gontarska, K. & Thamsen, L. Let’s wait awhile: how temporal workload shifting can reduce carbon emissions in the cloud. In Proceedings of the 22nd International Middleware Conference (Middleware ’21) (ACM, 2021). https://doi.org/10.1145/3464298.3493399

7. Riepin, I., Brown, T. & Zavala, V. M. Spatio-temporal load shifting for truly clean computing. Advances in Applied Energy 17, 100202 (2025). https://doi.org/10.1016/j.adapen.2024.100202

8. Luo, X. & Yang, C. AI inference as relocatable electricity demand: a latency-constrained energy-geography framework. Preprint at arXiv:2604.27855 (2026).

9. Cloudflare. Radar Documentation: HTTP Requests Timeseries (Cloudflare, 2025). https://developers.cloudflare.com/radar/

10. Global Energy Observatory, Google, KTH Royal Institute of Technology, Enipedia & World Resources Institute. Global Power Plant Database v1.3.0 (World Resources Institute, 2021). https://datasets.wri.org/datasets/global-power-plant-database

11. Ueckerdt, F., Hirth, L., Luderer, G. & Edenhofer, O. System LCOE: what are the costs of variable renewables? Energy 63, 61–75 (2013). https://doi.org/10.1016/j.energy.2013.10.072

12. Hirth, L., Ueckerdt, F. & Edenhofer, O. Integration costs revisited—an economic framework for wind and solar variability. Renewable Energy 74, 925–939 (2015). https://doi.org/10.1016/j.renene.2014.08.065

13. MacDonald, A. E. et al. Future cost-competitive electricity systems and their impact on US CO2 emissions. Nature Climate Change 6, 526–531 (2016). https://doi.org/10.1038/nclimate2921

14. Schlachtberger, D. P., Brown, T., Schramm, S. & Greiner, M. The benefits of cooperation in a highly renewable European electricity network. Energy 134, 469–481 (2017). https://doi.org/10.1016/j.energy.2017.06.004

15. Jiang, H. et al. Globally interconnected solar-wind system addresses future electricity demands. Nature Communications 16, 4523 (2025). https://doi.org/10.1038/s41467-025-59879-9

16. Davidson, M. R. & Millstein, D. Limitations of reanalysis data for wind power applications. Wind Energy 25, 1646–1653 (2022). https://doi.org/10.1002/we.2759

17. Sun, W. & Harrison, G. P. Wind-solar complementarity and effective use of distribution network capacity. Applied Energy 247, 89–101 (2019). https://doi.org/10.1016/j.apenergy.2019.04.042

18. Luthander, R., Widén, J., Nilsson, D. & Palm, J. Photovoltaic self-consumption in buildings: a review. Applied Energy 142, 80–94 (2015). https://doi.org/10.1016/j.apenergy.2014.12.028

19. Sartori, I., Napolitano, A. & Voss, K. Net zero energy buildings: a consistent definition framework. Energy and Buildings 48, 220–232 (2012). https://doi.org/10.1016/j.enbuild.2012.01.032

20. Weitzman, M. S. Measures of Overlap of Income Distributions of White and Negro Families in the United States. Technical Paper No. 22 (U.S. Bureau of the Census, 1970).

21. López Prol, J., de Llano-Paz, F., Calvo-Silvosa, A., Pfenninger, S. & Staffell, I. Wind-solar technological, spatial and temporal complementarities in Europe: a portfolio approach. Energy 292, 130348 (2024). https://doi.org/10.1016/j.energy.2024.130348

22. Ajagekar, A. & You, F. Variational quantum circuit learning-enabled robust optimization for AI data center energy control and decarbonization. Advances in Applied Energy 14, 100179 (2024). https://doi.org/10.1016/j.adapen.2024.100179

23. Liu, H., Brown, T., Andresen, G. B., Schlachtberger, D. P. & Greiner, M. The role of hydro power, storage and transmission in the decarbonization of the Chinese power system. Applied Energy 239, 1308–1321 (2019). https://doi.org/10.1016/j.apenergy.2019.02.009

24. Xing, J. et al. Carbon Responder: coordinating demand response for the datacenter fleet. Preprint at arXiv:2311.08589 (2023).

25. Wald, D., Johnson, K., King, J., Comden, J. & Bay, C. J. Shifting demand: reduction in necessary storage capacity through tracking of renewable energy generation. Advances in Applied Energy 10, 100131 (2023). https://doi.org/10.1016/j.adapen.2023.100131

26. Wang, Y. et al. BurstGPT: a real-world workload dataset to optimize LLM serving systems. Preprint at arXiv:2401.17644 (2024).

27. Microsoft Azure. Azure LLM Inference Trace 2024, in AzurePublicDataset (Microsoft, 2024). https://github.com/Azure/AzurePublicDataset/blob/master/AzureLLMInferenceDataset2024.md

28. National Renewable Energy Laboratory. Cost Projections for Utility-Scale Battery Storage: 2025 Update (NREL, 2025). https://www.nrel.gov/docs/fy25osti/93281.pdf

29. National Renewable Energy Laboratory. 2024 Annual Technology Baseline: Utility-Scale Battery Storage (NREL, 2024). https://atb.nrel.gov/electricity/2024/utility-scale_battery_storage

30. Kittel, M., Roth, A. & Schill, W.-P. Long-duration electricity storage needs for coping with Dunkelflaute events in Europe. Nature Communications 17, 4210 (2026). https://doi.org/10.1038/s41467-026-72681-5

31. U.S. Department of Energy. Long Duration Storage Shot (DOE, 2021). https://www.energy.gov/eere/long-duration-storage-shot

32. Sepulveda, N. A., Jenkins, J. D., Edington, A., Mallapragada, D. S. & Lester, R. K. The design space for long-duration energy storage in decarbonized power systems. Nature Energy 6, 506–516 (2021). https://doi.org/10.1038/s41560-021-00796-8

33. Schmidt, O., Hawkes, A., Gambhir, A. & Staffell, I. The future cost of electrical energy storage based on experience rates. Nature Energy 2, 17110 (2017). https://doi.org/10.1038/nenergy.2017.110

34. Ziegler, M. S. & Trancik, J. E. Re-examining rates of lithium-ion battery technology improvement and cost decline. Energy & Environmental Science 14, 1635–1651 (2021). https://doi.org/10.1039/D0EE02681F

35. Joskow, P. L. Comparing the costs of intermittent and dispatchable electricity generating technologies. American Economic Review 101, 238–241 (2011). https://doi.org/10.1257/aer.101.3.238

36. International Renewable Energy Agency. 24/7 Renewables: The Economics of Firm Solar and Wind (IRENA, 2026). https://www.irena.org/Publications/2026/May/24-7-renewables-The-economics-of-firm-solar-and-wind

37. Zippenfenig, P. Open-Meteo.com Historical Weather API (ERA5 archive). Zenodo (2023). https://doi.org/10.5281/zenodo.7970649

38. Hersbach, H. et al. The ERA5 global reanalysis. Quarterly Journal of the Royal Meteorological Society 146, 1999–2049 (2020). https://doi.org/10.1002/qj.3803

39. International Renewable Energy Agency. IRENASTAT Online Data Query Tool (IRENA, 2025). https://www.irena.org/Data

40. McKinsey & Company. The Cost of Compute: A $7 Trillion Race to Scale Data Centers (McKinsey, 2025).

41. Gartner. Gartner Says Electricity Demand for Data Centers to Grow 16% in 2025 and Double by 2030 (17 November 2025).

42. Kamiya, G. & Coroama, V. C. Data Centre Energy Use: Critical Review of Models and Results (EDNA-IEA 4E TCP, 2025). https://www.iea-4e.org/edna/publications/data-centre-energy-use-critical-review-of-models-and-results/

43. Patterson, D. et al. The Carbon Footprint of Machine Learning Training Will Plateau, Then Shrink. Computer 55, 18–28 (2022). Preprint at arXiv:2204.05149.

44. Electric Power Research Institute. Powering Intelligence: Analyzing Artificial Intelligence and Data Center Energy Consumption (EPRI, 2024).

45. Goldman Sachs Research. AI to Drive 165% Increase in Data Center Power Demand by 2030 (Goldman Sachs, 2025).

46. ABI Research. Forecasting Data Center Capacity Increases in the AI Era (ABI Research, 2026).

47. Pfenninger, S. & Staffell, I. Long-term patterns of European PV output using 30 years of validated hourly reanalysis and satellite data. Energy 114, 1251–1265 (2016). https://doi.org/10.1016/j.energy.2016.08.060

48. Staffell, I. & Pfenninger, S. Using bias-corrected reanalysis to simulate current and future wind power output. Energy 114, 1224–1239 (2016). https://doi.org/10.1016/j.energy.2016.08.068

49. Tong, D. et al. Geophysical constraints on the reliability of solar and wind power worldwide. Nature Communications 12, 6146 (2021). https://doi.org/10.1038/s41467-021-26355-z

50. Sterl, S. et al. Smart renewable electricity portfolios in West Africa. Nature Sustainability 3, 710–719 (2020). https://doi.org/10.1038/s41893-020-0539-0

51. Microsoft Azure. Azure network round-trip latency statistics between regions (Microsoft, accessed 2026).

52. Amazon Web Services. AWS Global Infrastructure: Regions and Availability Zones (AWS, accessed 2026).

53. Microsoft Azure. Azure geographies and regions (Microsoft, accessed 2026).

54. Google Cloud. Global locations—regions and zones (Google Cloud, accessed 2026).

55. Bundesnetzagentur. SMARD Electricity Market Data (Germany, accessed 2026). https://www.smard.de

56. Elexon. Balancing Mechanism Reporting Service (BMRS) generation data (United Kingdom, accessed 2026).

57. Form Energy. Iron-air battery technology and long-duration storage performance targets (Form Energy, accessed 2026).

58. Long Duration Energy Storage Council & McKinsey & Company. Net-zero Power: Long Duration Energy Storage for a Renewable Grid (LDES Council, 2021). https://www.ldescouncil.com/assets/pdf/LDES-brochure-F3-HighRes.pdf

59. Energy and Environmental Economics, Inc. Load Growth Is Here to Stay, but Are Data Centers? (E3, 2024).

60. Zhong, Y. et al. DistServe: Disaggregating Prefill and Decoding for Goodput-optimized Large Language Model Serving. In 18th USENIX Symposium on Operating Systems Design and Implementation (OSDI 2024). Preprint at arXiv:2401.09670.

61. Agrawal, A. et al. Taming throughput-latency tradeoff in LLM inference with Sarathi-Serve. In 18th USENIX Symposium on Operating Systems Design and Implementation (OSDI 24) 117-134 (USENIX Association, 2024). https://www.usenix.org/conference/osdi24/presentation/agrawal

62. Liu, J. et al. Andes: Defining and Enhancing Quality-of-Experience in LLM-Based Text Streaming Services. Preprint at https://arxiv.org/abs/2404.16283 (2024).

63. RAGO: Systematic Performance Optimization for Retrieval-Augmented Generation Serving. In Proceedings of ISCA 2025. Preprint at arXiv:2503.14649.

64. Cheney, D. How GitHub Copilot Serves 400 Million Completion Requests a Day. InfoQ (2025). https://www.infoq.com/presentations/github-copilot/

65. Google Research. ML-enhanced Code Completion Improves Developer Productivity (2022).

66. Semenkin, A. et al. Full Line Code Completion: Bringing AI to Desktop. Preprint at https://arxiv.org/abs/2405.08704 (2024).

67. LiveKit. Pipeline Types. LiveKit Documentation (accessed 13 July 2026). https://docs.livekit.io/agents/models/pipelines/

68. Ethiraj, V., David, A., Menon, S. & Vijay, D. Toward Low-Latency End-to-End Voice Agents for Telecommunications Using Streaming ASR, Quantized LLMs, and Real-Time TTS. Preprint at https://arxiv.org/abs/2508.04721 (2025).

69. Stivers, T. et al. Universals and cultural variation in turn-taking in conversation. Proceedings of the National Academy of Sciences 106, 10587–10592 (2009). https://doi.org/10.1073/pnas.0903616106

70. Google. Speed Matters for Google Web Search (Google, 2009).

71. Nielsen, J. Response Times: The 3 Important Limits. Nielsen Norman Group (1993). https://www.nngroup.com/articles/response-times-3-important-limits/ (accessed 13 July 2026).

72. SANA-Sprint: One-step Diffusion with Continuous-time Consistency Distillation. Preprint at arXiv:2503.09641 (2025).

73. Chen, Y.-H. et al. Speed Is All You Need: On-Device Acceleration of Large Diffusion Models via GPU-Aware Optimizations. Preprint at https://arxiv.org/abs/2304.11267 (2023).

74. AdaCache: Adaptive Caching for Faster Video Generation with Diffusion Transformers. Preprint at arXiv:2411.02397 (2024).

75. On-device Sora. Preprint at arXiv:2502.04363 (2025).

76. OpenAI. Batch API Guide (accessed 2026). https://developers.openai.com/api/docs/guides/batch

77. Anthropic. Message Batches API (accessed 2026). https://www.anthropic.com/news/message-batches-api

78. EcoServe: Designing Carbon-Aware AI Inference Systems. Preprint at arXiv:2502.05043 (2025).

79. Intercontinental round-trip-time measurements compiled from EXA Express, Southern Cross, RIPE Atlas/GeoCables and WonderNetwork; see also Preprint at arXiv:2303.02514 (2023).